|  | PROPOSED New baseline and Monitoring methodology oR methodological tool FORM FOR EMISSION REDUCTIONS ACTIVITIES (Version 02.0) |  |
| --- | --- | --- |
| Information to be completed by the secretariat and Methodologies Expert Panel |  |  |
| Type of standard |  | Choose an item. |
| Unique reference number and title of the proposed new methodology or new methodological tool |  | >> |
| Date when this form was received at UNFCCC secretariat: |  | Click or tap to enter a date. |
| Date of posting in the UNFCCC A6.4 web site for global stakeholder consultation |  | Click or tap to enter a date. |

Information to be completed by the submitter (read before filling the form)

This form is required at the “submission of proposed new methodology or methodological tool” stage and is submitted together with ‘New baseline and monitoring methodology and methodological tool proposal form (A6.4-FORM-METH-001).

Instructions for using this form In using this form, please follow the guidance established in the following documents: - Fill out all relevant sections of the form in clear print or typing; - Provide your input after the >> indicator in the space provided; - Leave blank sections which are found to be not applicable. Formatting Instructions: - Do not modify any part of this form, including headings, logo, format or font; - The form provides the formatted headings which should be used throughout the document; - Please use word equation editor to write equations; - Please format figures, tables and footnotes to update automatically; - Please note the footnotes have a separate format (Times New Roman - size 10). - Please clearly distinguish between proper methodology text, tables and equations and explanatory notes, using the following colour coding: - Methodology text shall be written in black fonts. - Guidance from the UNFCCC is provided in blue fonts and can be deleted. - Explanatory notes shall be written in grey fonts. Please note that explanatory notes are solely for the sake of methodology submission and consideration. Do not include guidance to activity participant in explanatory notes. Please note upon methodology approval, explanatory notes will be deleted.

- Summary and applicability of the baseline and monitoring methodology or methodological tool

Title, submission date and version

Title: Production/Manufacture of Low Carbon Intensive Ammonia Through Electrolysis of Water, Air Separation and Synthesis of Hydrogen and Nitrogen.

Submission date: 05th March 2025

Version: 1.0

Submission date: 21st April 2025

Version: 1.1

Submission date: 11th August 2025

Version: 1.2

Submission date: 24th September 2025

Version: 1.3

Submission date: 16th October 2025

Version: 1.4

Submission date: 20th November 2025

Version: 1.5

Submission date: 14th January 2026

Version: 1.6

Submission date: 23rd February 2026

Version: 1.7

If this methodology or methodological tool is based on a previous submission or an approved Article 6.4 mechanism methodology or methodological tool, please state the reference numbers here. Explain briefly the main differences and their rationale.

This methodology is not based on a previous submission or an approved PACM mechanism methodology

Summary description of the methodology or methodological tool, including major baseline and monitoring methodological steps.

This mechanism methodology is globally applicable to A6.4 project activities producing low carbon/green ammonia, whether implemented as retrofit or greenfield project activity. Emission reductions are primarily achieved through the use of renewable electricity across all ammonia production processes:

- Hydrogen production: Renewable electricity powers water electrolysis units, replacing fossil fuel-based hydrogen generation pathways such as steam methane reforming (SMR).

- Nitrogen production: Nitrogen is supplied via air separation units or equivalent systems that operate entirely on renewable energy.

- Ammonia synthesis: While the fundamental chemistry remains unchanged, all electricity required for synthesis, including compression, heating, pressurization, and auxiliary utilities, is sourced from renewable energy. Waste gas from the ammonia plant or syngas is used to fuel flares during normal operation, while LPG is used only as a backup fuel for pilot burners during shutdowns, plant trips, or load-limiting conditions to ensure safe flare operation.

The primary emission reductions arise from the switch from fossil fuel-based hydrogen production to renewable-powered electrolysis, making hydrogen production the key emission reduction driver in this methodology. However, the methodology ensures that all subprocesses of ammonia production, including nitrogen separation and synthesis, are powered exclusively by renewable energy.

Renewable electricity consumption in the ammonia production project activity is demonstrated through contractual allocation from identified renewable energy sources and reconciliation of time-stamped, time-block-wise metered electricity generation and project electricity consumption data, in accordance with the prevailing grid scheduling and settlement framework. This approach ensures that the renewable electricity generation allocated to the project activity meets or exceeds its electricity consumption during the monitoring period, without double counting.

Baseline scenario determination:

- For retrofitted projects:
The baseline scenario shall be determined using the existing actual or historical emissions corrected for downward adjustment.

- For greenfield projects:
The baseline scenario shall be determined using the BAT approach.

- Proposed new baseline and monitoring methodology or methodological tool

- Introduction

- The following table presents a summary of the key elements of a methodology:

Table 1. Methodology key elements

| Type of GHG mitigation measure(s) | Fuel/feedstock switch Technology switch GHG destruction GHG formation avoidance Engineered carbon dioxide removal Nature based carbon dioxide removal |
| --- | --- |
| Types of mitigation outcomes achieved under this methodology | Emission reductions Removals |
| Are the mitigation outcomes under this methodology at risk of reversal? | Yes No |
| Typical projects eligible under the methodology | Construction and operation of low carbon/green ammonia plants where all subprocesses are greenfield and powered by renewable energy, or projects where the electrolyser is greenfield and other subprocesses are existing or retrofitted, with the entire production process powered by renewable energy to enable low carbon/green ammonia production. |

- Scope and entry into force

Scope

This methodology applies to project activities that reduce greenhouse gas (GHG) emissions through the low carbon/green ammonia production by substituting conventional, fossil-fuel-intensive processes with processes powered by renewable electricity. The typical projects eligible under this methodology are those that integrate:

- Hydrogen production via electrolysis powered by renewable energy

- Nitrogen production through air separation using renewable electricity

- Ammonia synthesis through the Haber-Bosch process using the renewable energy.

The methodology is applicable to:

- Integrated retrofitted ammonia production projects, where a new greenfield electrolyser is installed for renewable hydrogen production, while existing subprocesses—such as air separation and ammonia synthesis—may be modified or upgraded to operate on renewable electricity.

- Integrated greenfield ammonia production projects, where all subprocesses are newly established (greenfield) to produce low-carbon/green ammonia using renewable-energy-driven processes.

Entry into force

- [For the UNFCCC secretariat to complete - Leave blank]

Applicability of sectoral scopes

Designated operational entities validating and verifying Article 6.4 activities that use this methodology shall apply sectoral scope(s):

- [For the UNFCCC secretariat to complete – leave blank]

- Definitions

In addition to the definitions contained in the ‘Article 6.4 mechanism Glossary of Terms’, the following definitions apply for the purpose of this methodology:

Air separation plants - A plant/process that separates atmospheric air into its primary components, typically nitrogen and oxygen, and sometimes also argon and other rare inert gases. Air separation plants are constructed in different forms depending on what products are produced, production capacity, and purity.

Ammonia (NH3) - Ammonia is a colourless gas that is used a feedstock in the production of fertilizers, chemicals and refrigerants.

Captive/third-party greenfield renewable power plants/storage assets - Renewable energy facilities or storage assets that are newly developed (greenfield) and specifically intended to supply electricity to the identified low-carbon/green ammonia production facility, developed in parallel with the ammonia production facility and commissioned to align with the start of its operations.

Captive/third-party newly constructed renewable power plants/storage assets - Renewable energy facilities or storage assets commissioned within the last 36 months, developed by the project proponent or a third party, specifically intended to supply electricity to the identified low-carbon/green ammonia production facility

Conventional ammonia production - Conventional ammonia production refers to the production of ammonia (NH₃) using fossil fuel–based processes, where hydrogen is primarily generated through steam methane reforming (SMR), coal gasification + water shift reaction or other fossil-fuel pathways, and nitrogen is obtained through air separation units powered by conventional energy sources (grid). This production route is characterized by significant CO₂ emissions resulting from both fuel combustion and processes.

Electrolyser hydrogen production plant - A facility that produces hydrogen from the electrolysis of water. Under this methodology, the hydrogen production plant includes the desalted water station, electrolytic cell, hydrogen compressor, gas-liquid processor, hydrogen purification unit and other ancillary equipment.

Gasification of coal - an industrial process where coal is converted into syngas through a process of gasification.

Low carbon/green Ammonia - Refers to ammonia (NH₃) produced using renewable energy sources for all upstream energy requirements, with only minimal and safety-related fossil fuel use permitted where unavoidable. The production process involves:

- Production of hydrogen (H₂) via electrolysers powered by renewable electricity, replacing fossil fuel-based hydrogen (e.g., from SMR).

- Production of nitrogen (N₂) through Air Separation Units or other facilities using renewable energy.

- Synthesis of hydrogen and nitrogen via the conventional Haber-Bosch process, where all electricity required for the reaction and ancillary utilities is supplied from renewable sources.

Green Hydrogen - Hydrogen (H₂) produced through the electrolysis of water, where the electricity used for electrolysis is derived exclusively from renewable energy sources.

Haber Bosch Process – It is the process of producing ammonia (NH₃) industrially by reacting nitrogen (N₂) from air with hydrogen (H₂) under high temperature and pressure in the presence of a catalyst. The process requires significant energy, which can be supplied by fossil fuels (e.g., natural gas, coal) or renewable sources (e.g., electricity from wind, solar, or hydro) for both hydrogen production and the ammonia synthesis step. When all energy inputs come from renewable sources, the process produces low carbon/green ammonia when fossil fuels are used, it produces grey ammonia.

Retrofitted low carbon/green ammonia production project activity: The electrolyser plant for hydrogen production is implemented as a greenfield unit, while other subprocesses—such as air separation and ammonia synthesis—are existing facilities that may be retrofitted or partially modified to operate on renewable electricity

Steam methane reforming (SMR) process - The steam reforming process for ammonia production uses natural gas to produce hydrogen, which is then combined with nitrogen to create ammonia. It is the process through which hydrogen is generated from natural gas. High-temperature steam ranging between 700°C–1,000°C is used to produce hydrogen from a methane source, such as natural gas. It is to be noted that the process is endothermic, i.e., heat must be supplied for the initiation of the reaction.

Steam reforming of natural gas - an industrial process where natural gas reacts with steam in the presence of a catalyst to produce hydrogen, carbon monoxide and releases carbon dioxide as a by-product.

Steam reforming of syngas - an industrial process where the syngas produced from the gasification of coal or oil reacts with steam in the presence of a catalyst to produce hydrogen, carbon monoxide and releases carbon dioxide as a by-product.

Water-gas shift - A chemical reaction in which carbon monoxide (CO), derived from syngas, reacts with steam (H₂O) to produce carbon dioxide (CO₂) and hydrogen (H₂).

- Normative references

This proposed baseline and monitoring methodology is based on the following proposed new methodologies and/or approved or consolidated methodologies:

Feed switch in integrated Ammonia-urea manufacturing industry”. AM0050 / Version 03.0.0/ Sectoral scope: 05/EB 68

Hydrogen production from electrolysis of water, AM0124 / Version 1.1 / Sectoral scope: 1,5 / EB 119

This methodology also refers to the latest approved versions of the following methodological tools/standard:

A6.4-STAN-METH-005 – Addressing leakage in mechanism methodologies (v.01.0)

A6.4-STAN-METH-001 – Application of the requirements of Chapter V.B (Methodologies) for the development and assessment of Article 6.4 mechanism methodologies (v.01.1)

A6.4-AMT-001: Common practice analysis” (hereinafter referred as “common practice tool”).

A6.4-AMT-002: Investment analysis” (hereinafter referred as “investment analysis tool”).

A6.4-STAN-METH-004 - Setting the baseline in mechanism methodologies (v.01.0)

This methodology is based on the following sources of information:

Ammonia production processes from energy and emissions perspectives: A technical brief

Product carbon footprint methodology for ammonia production by conventional steam reforming–A case study: European Journal of Sustainable Development Research 2024, 8(1), em0241 e-ISSN: 2542-4742

2019 refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories /Section 3.2 / Chapter 3: Chemical Industry Emissions

Techno-Economic Challenges of Green Ammonia as an Energy Vector / Chapter 4: Ammonia Production Technologies

- Applicability

The methodology is applicable under the following conditions:

  - This methodology applies to project activities that fall under one of the following scenarios:

- (i) Integrated retrofitted low carbon/green ammonia production project activity: The electrolyser plant for hydrogen production is implemented as a greenfield unit, while other subprocesses—such as air separation and ammonia synthesis—are existing facilities that may be retrofitted or partially modified to operate on renewable electricity

- (ii) Integrated greenfield low carbon/green ammonia production project activity: All subprocesses involved in ammonia production are greenfield

  - In the case of a retrofitted A6.4 project activity, where the electrolyser plant is greenfield and all subprocesses are existing or retrofitted, the output(s) or level(s) of service must remain identical to those in the pre-A6.4 project condition.

  - This methodology is limited to the production and storage of ammonia within the manufacturing facilities.

  - Renewable energy for the ammonia production process shall be sourced either from greenfield captive/third-party renewable power plants/storage assets, OR from newly constructed captive/third-party renewable power plants/storage assets commissioned within the last 36 months with the explicit intention of supplying electricity to the specific low carbon/green ammonia production facility.

  - Electricity from third-party renewable power plants/storage assets shall be supplied either via a dedicated transmission line or through the grid under a Power Purchase Agreement (PPA) to ensure traceability.

  - The project proponent shall demonstrate that greenfield captive/third-party renewable power plants/storage assets or newly constructed captive/third-party power plants/storage assets are directly linked to the low carbon/green ammonia production facility. Compliance with this requirement shall be verified at the time of validation.

  - The project proponent shall demonstrate temporal and volumetric traceability of renewable electricity consumption by the low carbon/green ammonia production facility. Electricity generation from captive/third-party renewable power plants/storage assets and electricity consumption by the ammonia production facility shall be monitored using separate, calibrated, and auditable smart electricity meters installed, as applicable, at the renewable electricity generation source, grid interconnection point, storage assets (charging and discharging), and the ammonia production facility.

  - Metered data shall be reconciled at a defined time resolution (e.g., 15-minute or hourly) to demonstrate that renewable electricity from contractually allocated and identified renewable energy sources, allocated to the ammonia production project activity, is available and sufficient to meet or exceed electricity consumption during the monitoring period, ensuring temporal and volumetric matching.

  - Where the green ammonia facility draws electricity from multiple renewable energy sources, the final drawl or consumption schedules from each source shall be used to allocate the facility’s total electricity consumption accordingly.

  - Where renewable electricity is sourced from captive or third-party renewable power plants that also supply electricity to other end users through the grid, the project proponent shall demonstrate exclusive allocation of renewable electricity and associated environmental attributes (e.g., renewable energy certificates or equivalent instruments) to the project activity, and shall ensure that such attributes are not retained, sold, or claimed by any other entity.

  - The project activity shall ensure that, on an annual basis, the ratio of electricity consumed from the grid to the electricity consumed from the captive greenfield or newly constructed captive/third-party renewable power plant for the ammonia production facility remains below 0.1. The Designated Operational Entity (DOE) shall verify compliance with this requirement by annually comparing the monitored electricity consumption data from both sources.

  - The project shall limit the use of locally available water (whether surface or groundwater) for electrolysis to no more than 5%, ensuring minimal impact on water needed for drinking, agriculture, or livelihoods. Compliance with this requirement will be verified during validation and at each crediting period renewal using project data and official sources.

  - If the project activity involves installing a greenfield electrolyser within an existing conventional ammonia production facility, the existing equipment used for hydrogen production must be decommissioned, destroyed, or properly disposed of. Failure to do so will require that emissions associated with its continued use be fully accounted for as per the section 12.3, para 101. The Designated Operational Entity (DOE) shall be responsible for verifying compliance with this requirement.

  - Only the quantity of ammonia that is produced using renewable hydrogen and nitrogen inputs, and is verifiably exported or sold, shall be considered eligible for emission reduction calculations. Supporting evidence may include sales invoices, dispatch records, or delivery logs.

  - Furthermore, the project activity shall meet all applicability conditions defined under the applicable methodological tools.

- Avoidance of double-counting

All activity participants shall demonstrate that the A6.4 activity will not result in double counting by:

Providing evidence, in each monitoring report, that the outcomes from the A6.4 activity (e.g. ammonia production) for which they intend to request issuance of A6.4ERs are not also claimed in other environmental markets or accounting framework (e.g. guarantees of origin for renewable energy generation, green hydrogen schemes, low-carbon fuel standards), except for outcomes not related to reducing greenhouse gases emissions (e.g., air contaminant reductions or social impacts); and

Demonstrating that the reported GHG emission reductions for which they intend to request issuance of A6.4ERs do not overlap with mandatory domestic mitigation schemes (e.g., emissions trading systems), or that measures are in place to ensure that any relevant impacts of the activity (e.g. the GHG emission reductions achieved or the kilowatt-hours of renewable electricity produced) are not counted towards the achievement of targets or obligations under the mandatory domestic mitigation scheme (e.g. by cancelling allowances from the emissions trading system before issuing carbon credits) if the overlap exists, by:

Declaring and providing evidence in each monitoring report that the A6.4 activity do not fall within the scope of any mandatory domestic mitigation scheme: or

Where the A6.4 activity fall within the scope of a mandatory domestic mitigation scheme, activity participants may:

Provide evidence in each monitoring report that the mitigation outcomes of the A6.4 activity are not counted in the mandatory mitigation scheme to reduce the obligations by the entities covered by the scheme.

Demonstrate that project participants are not requesting the issuance of A6.4ERs for any emission reductions resulting from a component of the A6.4 activity that falls within the scope of the mandatory domestic scheme.

A6.4 activities that involve the use of ammonia by consumers other than the activity participants shall also employ signed contracts with all consumers of the ammonia produced under the A6.4 activity. These contracts must explicitly indicate that A6.4ERs can only be claimed by the activity participants and that the users of the low carbon/green ammonia produced shall not claim A6.4ERs or carbon credits from any other carbon crediting programme.

Notwithstanding paragraph ‎9 above, where the policy for establishing the framework or environmental market or for establishing the mandatory domestic mitigation scheme refers to or formally integrates the mechanism as an instrument for implementation, participation in such a framework or environmental market or domestic mitigation scheme does not result in double counting.

- Demonstration of alignment with the policies, options and implementation plans with regard to the NDC and LT-LEDS of the host Party and the long-term temperature goal of the Paris Agreement and long-term goals of the Paris Agreement

Activity participants shall attach to the PDD presented to the DOE performing the validation a confirmation from the DNA of the host Party that the DNA has undertaken an assessment of the activity’s consistency with Decision 3/CMA.3 paragraph 40 (c) and paragraph 27 (a), as part of the host Party’s approval, to demonstrate that the activity does not constrain, but aligns with the policies, options and implementation plans of the host Party with regard to the nationally determined contribution (NDC) of the host Party, its long-term low greenhouse gas emission development strategies (LT-LEDS) if it has submitted one, and the long-term temperature goal of the Paris Agreement and long-term goals of the Paris Agreement.

- Activity Boundary

The project boundary includes:

- The specific location and grid from which renewable electricity is generated and supplied.

- The location where the hydrogen production facility consisting of electrolyser stacks; water desalination/treatment and pumping.

- The location where the air separation plants are located to produce nitrogen.

- The location where the hydrogen is combined/ synthesised with nitrogen to produce the low carbon/green ammonia.

- The boundary of the project activity is limited to the premises of production facility(ies) and the supply sources of renewable energy.

Table 2. Emissions sources and sinks included in or excluded from the activity boundary

|  | - Source | - GHG |  |  | - Justification / Explanation |
| --- | --- | --- | --- | --- | --- |
| - BASELINE | - Ammonia Production, excluding transportation | - CO2 | Included - Not included | Controlled Related to - Affected by | - Main emission source |
|  |  | - CH4 | Included - Not included | Controlled Related to - Affected by | - Minor emission source |
|  |  | - N2O | Included - Not included | Controlled Related to - Affected by | - Minor emission source |
|  |  | - ----- | Included - Not included | Controlled Related to - Affected by | - Not Applicable |
|  | - Hydrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Nitrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
| - ACTIVITY | - Low carbon intensive ammonia production, excluding transportation. | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Hydrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Nitrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
| - LEAKAGE | - Transferred baseline equipment | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Resource competition | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Diversion of production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Indirect environmental emissions | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |

- Demonstration of additionality

Regulatory analysis

The activity participant of the project shall demonstrate that the mitigation activity is not a legal requirement. However, the activity participant shall ensure that all the rules and regulations related to the ammonia production process is mandatorily adhered to. The statutory documents (Environmental Impact Assessment, license to operate and other terms and conditions etc) shall be made available during the validation process.

The activity participant shall obtain the Host Party approval (through its DNA), If the host Party approves the project, the approval shall include:

- Confirmation that, and information on how, the project fosters sustainable development in the host Party.

- Approval of any potential renewal of the crediting period, if the Party intends to allow the project to continue beyond the first crediting period, where the Party has specified that the crediting periods of A6.4 activities that it intends to host may be renewed pursuant to paragraph 27(b) of the RMPs;6

- Explanation of how the project relates to the implementation of its NDC and how the expected GHG emission reductions or net GHG removals contribute to the host Party’s NDC, and the purposes referred to in Article 6, paragraph 1.

- Authorization of the activity participants

Avoidance of locking-in the level of emissions

A paragraph Locking-in emissions refers to project activities that commit process to a high-emission pathway (e.g., fossil fuel infrastructure with long asset lifetimes), making future decarbonization harder. Ammonia production process, based on electrolysis powered by dispatchable RE, inherently avoids lock-in. However, to rigorously demonstrate this, the following process may be applied:

Definition of Lock-In Emissions Criteria

- Lock-in risks are identified by assessing whether the project:

- Creates dependency on fossil fuels (e.g., requires fossil-based backup systems).

- Extends the lifespan of high-emission infrastructure (e.g., retrofits coal plants instead of replacing them).

- Adopts technologies with no decarbonization pathway (e.g., irreversible design choices).

- Triggers market/regulatory shifts favouring fossil fuels (e.g., crowding out renewables).

Process to Identify Lock-In Emissions Risks

- Step 1: Project Component Analysis

| Project Component | Lock-In Risk Assessment | Required Mitigation and Evidence |
| --- | --- | --- |
| Electrolysis Technology | If designed with fossil-based backup (e.g., grid-tied without RE guarantee), may create long-term reliance on fossil grid power.. | Electrolysers are RE-powered and modular. Submit technical specifications and supplier declarations showing 100% RE use. |
| Air Separation Unit (ASU) | If powered by fossil grid or installed with fossil backup systems, it may lock in carbon-intensive nitrogen production over time.. | Use RE-powered ASU only. Provide energy consumption breakdown and evidence of RE source. Confirm that no fossil-fired backup compressors are installed. |
| Haber-Bosch Ammonia Synthesis | Traditionally optimized for fossil-based hydrogen; risk of integrating fossil fuel preheating or steam inputs. | PFD (process flow diagram) showing no fossil heating or steam usage. Specify energy source for reactor heat integration (e.g., electrical or RE-derived thermal). |
| Dispatchable RE Source | If backed by diesel or grid-based fossil electricity during non-RE generation hours, may lock in backup fossil usage | Provide energy generation profiles and dispatch strategy demonstrating 24/7 RE-based operation. Include storage design documentation. |
| Grid Interconnection | If connected to a fossil-dominated grid without a clean energy contract or tracking, it may lead to indirect fossil fuel reliance. | Submit proof of exclusive RE supply through PPAs or wheeling agreements. Demonstrate no net fossil imports. |
| Auxiliary Systems | If auxiliary compressors/pumps rely on fossil fuels, long-term carbon emissions could be locked in.. | Submit equipment specifications showing use of electrically driven, zero-emission systems only. |

Step 2: Qualitative Checklist for Project Developers

The methodology includes the following checklist to be completed and substantiated with evidence during validation:

| Criterion | Requirement | Evidence Type |
| --- | --- | --- |
| No fossil fuel combustion in any process | Required | Process flow diagram, energy input data |
| Modular and upgradable RE infrastructure | Required | Technical specs, supplier declarations |
| No long-term fossil input contracts | Required | Fuel/power procurement contracts |
| Alignment with host country’s decarbonization pathway | Required | Policy documents, official targets |

Investment analysis, Barrier analysis and Common practice analysis

Investment analysis

To demonstrate additionality in accordance with the A6.4-STAN-METH-003 standard, this methodology requires the demonstration of financial additionality using investment analysis to assess whether the A6.4 activity is not financially viable in the absence of revenues from A6.4ERs.

Project proponents shall undertake a detailed financial assessment to demonstrate that the proposed activity is not financially attractive in the absence of carbon revenue from the sale of A6.4 emission reductions (A6.4ERs), and that such revenue is instrumental in the investment decision.

The methodology requires that the financial analysis be conducted based on data and assumptions consistent with those used in investment decision-making, reflecting conditions at the start date of the activity.

Type of Investment Analysis: The methodology requires activity participants to apply one of the following types of investment analysis as suitable to the project context, in line with paragraphs 34–36 of A6.4-STAN-METH-003:

- Benchmark analysis: comparing the financial viability of the project to a conservative financial benchmark such as the Weighted Average Cost of Capital (WACC).

- Investment comparison analysis: comparing the project’s financial attractiveness to alternative options.

- Simple cost analysis: for cases where the activity generates no financial return other than from A6.4ERs.

The type of analysis shall be justified by the activity participants, based on the nature of the activity (e.g., whether alternatives are available, whether it is a modification to an existing facility, or a greenfield investment).

Financial Indicators and Carbon Revenue Impact: The financial analysis shall calculate appropriate financial indicators (e.g., IRR, NPV, payback period), both with and without the expected carbon revenues from A6.4ERs. The results must clearly show that:

- The project is not financially viable without A6.4ER revenue (e.g., IRR below benchmark or NPV negative); and

- The inclusion of carbon revenue materially improves the financial viability, such that it becomes attractive to investors.

This comparison is essential to demonstrate that the carbon finance is a decisive factor in moving forward with the investment.

Data Requirements and Transparency: All financial data (e.g., capital investment, operational and maintenance costs, electricity/fuel revenues, carbon price assumptions, discount rates) must be based on publicly available sources, feasibility studies, or actual investment documents. These shall be transparently documented, with all key assumptions disclosed.

Where relevant, a sensitivity analysis shall be conducted on key variables (e.g., carbon price, input cost, product price) to assess the robustness of the additionality claim.

Barrier analysis

Barrier analysis is not applicable in this case, as the low carbon/green ammonia project does not relate to activities outlined in paragraphs 52 and 53 of A6.4-STAN-METH-004 - such as those implemented at individual households or by small entities without access to commercial or public third-party finance.

Common practice analysis

This methodology applies the Common Practice Assessment Tool (A6.4-MEP07-A01) under the PACM Mechanism to determine whether the proposed activity is common practice in the applicable geographical area to assess the additionality of the mitigation project activity.

The following parameters shall guide the implementation of the common practice analysis:

| Parameter | Guidance / Value | Justification of the Choice |
| --- | --- | --- |
| Approach used | Approach B – Target market size and market penetration | Approach B is selected because the ammonia sector lacks enough directly comparable low carbon/green ammonia facilities, which limits the effectiveness of Approach A. Approach B allows quantifying the diffusion of the low carbon/green ammonia production practice within the defined market. |
| Indicator type | Capacity/output-based – Annual ammonia production (tonnes/year), considering either installed design capacity or actual output. | Ammonia production is best measured through capacity or actual output, as it is directly tied to project scale and performance. This aligns with available industry data and regulatory reporting norms. |
| Data approach | Stock-based – All operational facilities started commercial operation before the project design document (A6.4-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity. | A stock-based approach ensures a comprehensive assessment of existing operational facilities at the time of project planning, representing the real state of technology deployment prior to the proposed project activity. |
| Geographical area | Should be the entire host country. If the project participants opt to limit the applicable geographical area to a specific geographical area (such as province, region, etc.) within the host country, then they shall provide justification on the essential distinction between the identified specific geographical area and rest of the host country. | Applying the analysis at the national level provides a holistic view of technology penetration and avoids regional data bias. If projects apply a sub-national scope, they must justify how the selected area is distinct in terms of market structure, infrastructure, or policy support. |
| Capacity/output range | Include projects with a production capacity or output within ±50% of the total design capacity or output of the proposed project activity. | The ±50% range aligns with the guidance provided in the Tool and ensures inclusion of comparable-scale projects |
| Common practice threshold (Fmax) | 20% – Default value as per the Common Practice Tool | Although the Common Practice Tool prescribes a default threshold of 3% for Approach B, a threshold of 20% is adopted to reflect the early-stage market of low carbon/green ammonia. Low carbon/green ammonia production is currently in a nascent phase globally, with limited diffusion due to high costs, lack of infrastructure, and policy uncertainty. Using a higher threshold (20%) provides a more meaningful assessment of whether technology is genuinely widespread. This adjustment aligns with the tool’s allowance for deviation when justified by sectoral characteristics and enhances environmental integrity by avoiding over-crediting of technologies. |
| Market penetration calculation | Share of low carbon/green ammonia production capacity relative to total ammonia capacity within the defined geographical area | Market penetration is best captured through the share of low carbon/green ammonia capacity in total ammonia production, as this clearly distinguishes the proposed low-emission technology from conventional ammonia production (e.g., SMR, coal gasification). |
| Validation responsibility | DOE to validate all assumptions, parameter selection, and data sources at validation stage and at each crediting period renewal | As per Tool guidance, Designated Operational Entities (DOEs) are required to validate all methodological applications. |

The outcome of the common practice analysis, conducted as per the guidance in this methodology and using Tool A6.4-MEP07-A01, shall be used to determine whether the proposed project activity is a widely adopted practice in the applicable geographical area. If the calculated market penetration of the proposed technology, measure, or practice is below the common practice threshold (Fmax), the activity shall be deemed as not common practice and may be considered additional under the PACM Mechanism.

The A6.4 activity is only additional if it is demonstrated that:

- (a) Emission reductions resulting from an A6.4 activity would not occur as a result of the legal requirements, per section 9.1 above; and

- (b) The A6.4 activity is not financially viable, per section 9.3.1 above; and

- (c) The A6.4 activity is not common practice, per section 9.3.3 above.

- Baseline scenario

Selection of the baseline approaches from paragraph 36 of the rules, modalities and procedures

The approach chosen to establish the baseline for ammonia production projects eligible under the mechanism methodology, is

For A6.4 activities implemented as retrofitted low carbon/green ammonia production project, the baseline is determined through existing actual or historical emissions (paragraph 36(iii) of the RMPs) because data from existing plant is strongly heterogeneous and emissions are highly activity- or site-specific).

For A6.4 activities implemented as greenfield low carbon/green ammonia production project, the baseline is determined based on the best available technology (BAT) approach, that represents an economically feasible and environmentally sound course of action (paragraph 36(i) of the RMPs).

Application of the selected approach, prior to implementation of a downward adjustment

Procedure for the identification of the baseline scenario

The baseline scenario for the A6.4 activities eligible under the mechanism methodology are described in the table below:

- Table 1: Baseline scenario for A6.4 activities eligible under the mechanism methodology

| - Component | - Baseline approach | - Baseline scenario |
| --- | --- | --- |
| - Ammonia production project activity – A6.4 activities implemented as retrofitted projects | - Existing actual or historical emissions | - The continuation of the preactivity scenario, i.e., conventional ammonia production. |
| - Ammonia production project activity – A6.4 activities implemented as greenfield project. | - Best Available Technologies that represent an economically feasible and environmentally sound course of action | - Determined based on sections 10.2.2.1 below |

- 10.2.2. Identification of the baseline scenario for A6.4 activities implemented as greenfield project activity

Activity participants shall determine the best available technology and economically feasible and environmentally sound course of action for A6.4 activities implemented as greenfield project activities based on the stepwise process described below.

- 10.2.2.1. Step 1: Identification of the baseline geographical reference area

The baseline geographical reference area is the host country by default.

Activity participants may choose to limit the baseline geographical reference area to a narrower specific geographical area (such as municipality, province, states, etc.) within the host country if they can demonstrate and justify that there are essential distinctions between the identified specific geographical area and rest of the host country. For example, such distinctions may include certain benefits that make the A6.4 activity financially/economically more attractive (e.g., subsidies or other financial flows), different climatic conditions.

- 10.2.2.2. Step 2. Identification of technologies and/or practices available in the baseline geographical reference area

Activity participants shall identify the technologies and/or practices, that are available for ammonia production in the baseline geographical reference area, and which can deliver the same production capacity.

Available technologies and/or practices are those that are accessible off the shelf, via a tendering or direct contracting process, or by direct implementation by an end user within the boundary of potential A6.4 activities

Technologies and/or practices that may be applied in ammonia production include, but are not limited to:

- (a) Primary hydrogen production pathways for ammonia synthesis, including:

- (1) Steam methane reforming (SMR) using conventional natural gas feedstock;
(2) Coal, oil/ or biomass-based gasification routes for hydrogen production;
(3) Electrolysis-based hydrogen production using grid electricity.

- (4) Electrolysis-based hydrogen production using renewable electricity

- (b) Emission reduction and mitigation measures, which may be implemented as add-on measures to the primary production pathways, including:

- (5) Application of carbon capture and storage (CCS) in hydrogen or conventional ammonia production;
(6) Hybrid approaches combining renewable energy with conventional processes to reduce emissions intensity.

- 10.2.2.3. Step 3. Identification of the environmentally sound technologies

Out of the technologies and/or practices identified in Step 2 above, activity participants shall exclude from the analysis those that do not comply with legal and regulatory requirements in the applicable baseline geographical area. For example, this may apply to ammonia production facilities that are not in compliance with limits for air emissions (e.g., NOx, SOx, CO₂), water discharges, hazardous waste handling, safety standards, or other environmental and occupational regulations.

- 10.2.2.4. Step 4. Identification of the economically viable technologies

Out of the technologies and/or practices that are identified as environmentally sound in Step 3 above, activity participants shall further identify those that are deemed economically viable (i.e., those that provide sufficient returns to cover investment, operations, and maintenance costs).

Technologies and/or practices are deemed economically viable if they:

- (a) Have been implemented under prevailing market conditions without significant subsidies by at least one facility within the applicable baseline geographical reference area without the revenues from carbon credits or other incentives; or

- (b) Are accessible off the shelf or via tendering within the applicable baseline geographical reference area, on terms that are economically viable when compared against a benchmark.

To identify the economic viability of a technology or practice under paragraph 45(b), activity participants shall conduct an investment analysis (using benchmark analysis approach) following the same requirements used for the additionality demonstration as per section 9.3.1 above and shall:

- (a) Collect relevant information directly from technology or practice suppliers; or

- (b) Demonstrate that it is not possible to collect relevant information directly from technology or practice suppliers. In this case, activity participants shall:

- (i) Where direct cost information from local suppliers is unavailable, cost and financial parameters may be derived from benchmarks, historical data from similar facilities, peer-reviewed literature, country-specific cost reports, or internationally recognized studies/standards. Clear justification shall be provided to demonstrate that such information reasonably reflects conditions within the applicable baseline geographical reference area. The DOE shall cross-verify this during validation and project registration.

- (ii) Provide proper justification that any such information can reasonably be applied in the baseline geographical reference area. The DOE shall cross-verify the same during the validation and project registration stage.

- 10.2.2.5. Step 5. Define the emissions intensity of each of the remaining technologies

Out of the economically viable technologies and/or practices identified in Step 4, activity participants shall determine the emissions intensity per tonne of ammonia production and shall:

- (a) Collect relevant information directly from technology suppliers; or

- (b) Demonstrate that it is not possible to collect relevant information directly from technology suppliers. In this case, activity participants:

- (i) May determine the emissions intensity from similar facilities located outside of the baseline geographical reference area, or from peer reviewed studies and papers; and

- (ii) Shall provide proper justification that any such information can reasonably be applied in the baseline geographical reference area.

- 10.2.2.6. Step 6. Selection of the Best Available Technology

The technology that constitutes the BAT is the one with the lowest emission intensity among those that are economically feasible and environmentally sound.

Calculation of baseline emissions prior to downward adjustment

Baseline emissions calculations, that applies approach based on the Best Available Technologies (BAT) for ammonia production, the following equations can be applied:

- BEy   =   BEBAT,y =  PNH3,y* SEBAT                       Equation (1)

With:

PNH3,y= QNH3,total,y                                         Equation (2)

Where:

| BEy | = | Baseline emission associated with the ammonia production project activity in year y (t CO2e/year) |
| --- | --- | --- |
| BEBAT,y | = | Baseline CO2 emissions associated with BAT in year y (t CO2e/year) |
| SEBAT | = | Specific emissions of the best available ammonia production technology (t CO2e/tNH3) |
| PNH3,y | = | Quantity of low carbon/green ammonia produced in PACM project activity in year (tNH3) |
| QNH3,total,y | = | Total quantity of low carbon/green ammonia produced and verifiably exported by the project activity in the year y (tNH3) |

Baseline emissions calculations that apply an approach based on existing actual or historical emissions for ammonia production may be determined using the following equations.

BEy   =    BEhistorical,y= PNH3,y * SEhistorical           Equation (3)

Where:

| SEhistorical​ | = | Historical average specific emission intensity from conventional ammonia production project activity (tCO₂/t NH₃) |
| --- | --- | --- |

With

SEhistorical​= t=1TECO2,y​t=1TPNH3,historical,y​                             Equation (4)

Where:

| SEhistorical​ | = | Historical average specific emission intensity from ammonia production project activity (tCO₂/t NH₃) |
| --- | --- | --- |
| ECO2,y | = | Actual CO2 emissions from historical ammonia production in year y (tCO₂) |
| PNH3,historical,y | = | Annual historical ammonia production from the conventional ammonia production facility during the pre-A6.4 project period in year y (tNH₃) |

For determining ECO2,yand PNH3, historical,y, in equation (4) above historical data from the existing ammonia production plant in which the new A6.4 activity will be implemented shall be used. This shall include data for the three years from the most recent operating history prior to the retrofit and before operations resumed, such as annual ammonia production volumes and emissions intensity per tonne of output. If data for fewer than three years but at least one year are available, the average of the available years shall be applied.

Calculation of the downward adjusted baseline

The downward adjusted baseline emissions are separately determined for the calendar year of the start date of the first crediting period and subsequent calendar years

- 10.3.1. Downward adjusted baseline emissions in the calendar year of the start date of the first crediting period

- 10.3.1.1. Step 1. Determine whether and to which components of the A6.4 activity an initial downward adjustment applies

An initial downward adjustment does not apply (DAi,y1 = 0) to the greenfield ammonia production project activities

For the greenfield low carbon/green ammonia project activity, the unadjusted baseline emissions, as determined in section 10.2.2 through equations (1) and (2), shall be used as the downward adjusted baseline emissions in the calendar year of the start date of the crediting period.

In retrofitted low carbon/green ammonia project activity instances, Steps 2 to 5 below shall be applied to determine the downward adjusted baseline emissions.

- 10.3.1.2. Step 2. Determine the uncertainty at the lower bound of the uncertainty interval (UNCBE act/hist,y1)

- Activity participants shall determine the overall uncertainty of baseline emissions at the lower bound of the 95 per cent confidence interval relative to the central estimate of estimated baseline emissions in the first year of the crediting period (UNCBE act/hist,y1).

- Activity participants shall consider uncertainty in all parameters used to calculate baseline emissions, following the guidance from Volume 1, Chapter 3 of the IPCC (2019 Refinement) through the error propagation method or Monte Carlo simulation.

- Activity participants shall document the uncertainties assigned to each parameter, specify assumptions made, and include any associated references for estimating the uncertainties of parameters in the PDD. This documentation and the derivation of the overall uncertainty through the error propagation or Monte Carlo simulation methods shall be provided together with the PDD (e.g., in a spreadsheet).

- 10.3.1.3. Step 3. Determine the downward adjusted baseline emissions based on uncertainty (BEadj,UNC,y1)

- For the calendar year of the start date of the first crediting period, determine the downward adjusted baseline emissions based on uncertainty (BEadj, UNC,y1) as follows:

| BEadj,UNC,y1=BEact/hist,y1×1-UNCBE act/hist,y1 | Equation (5) |
| --- | --- |

Where:

| BEadj, UNC,y1 | = | Downward adjusted baseline emissions based on uncertainty in year 1 of the crediting period (tCO2e/year) |
| --- | --- | --- |
| BEact/hist,y1 | = | Unadjusted existing actual or historical net baseline emissions and/or removals in year 1 of the crediting period (tCO2e/year) |
| UNCBEact/hist,y1 | = | Uncertainty of baseline emissions at the lower bound of the 95 per cent confidence interval relative to the central estimate of quantified unadjusted net baseline emissions in year 1 of the crediting period (fraction) |
| y1 | = | Calendar year of the start date of the crediting period |

- 10.3.1.4. Step 4. Determine the minimum downward adjusted baseline emissions (BEadj,min,y1)

- The minimum downward adjusted baseline shall be determined during the first year of the crediting period as follows:

| BEadj,min,y1=BEact/hist,y1-BEact/hist,y1-PEy1×0.1 | Equation (6) |
| --- | --- |

Where:

| BEadj,min,y1 | = | Minimum downward adjusted baseline emissions in year y (tCO2e/year) |
| --- | --- | --- |
| BEact/hist,y1 | = | Unadjusted existing actual or historical net baseline emissions in year 1 of the crediting period (tCO2e/year) |
| PEy1 | = | Project emissions in year 1 of the crediting period (tCO2e/year) |
| y1 | = | Calendar year of the start date of the first crediting period |

- The project emissions in referred in equation (17) and calculated as per section ‎11.1 below implemented under the A6.4 activity.

- 10.3.1.5. Step 5. Compare BEadj,UNC,y1 and BEadj,min,y1

- For each baseline emission component or sub-component i, the downward adjusted baseline emissions shall be determined as follows:

| BEadj,y1=minBEadj,min,y1 ; BEadj,UNC,y1 | Equation (7) |
| --- | --- |

Where:

| BEadj,y1 | = | Downward adjusted baseline emissions in year 1 of the crediting period (tCO2e/year) |
| --- | --- | --- |
| BEadj,min,y1 | = | Minimum downward adjusted baseline emissions in year 1 of the crediting period (tCO2e/year) |
| BEadj,UNC,y1 | = | Downward adjusted baseline emissions based on uncertainty in year 1 of the crediting period (tCO2e/year) |
| y1 | = | Calendar year of the start date of the crediting period |

- 10.3.1.6. Step 6. Determine the initial downward adjustment DAiy1

- The minimum downward adjustment and the downward adjustment based on the uncertainty are implicitly included in equations (5) and (6) above. The minimum downward adjustment is represented by the term (BEact/hist,y1 – PEy1) x 0.1. The downward adjustment based on the uncertainty is represented by the term (BEact/hist,y1 x (1-UNCBE act/hist,y1)). In the following section on downward adjusted baselines in subsequent years, the initial downward adjustment is required as a term. It shall be determined as the highest value between the minimum downward adjustment and the downward adjustment based on the uncertainty, as follows:

| DAy1=maxBEact/hist,y1-PEy1×0.1orBEact/hist,y1×UNCBEact/hist,y1 | Equation (8) |
| --- | --- |

- Where:

| DAy1 | = | Initial downward adjustment (t CO2e/year) |
| --- | --- | --- |
| BEact/hist,y1 | = | Unadjusted existing actual or historical net baseline emissions in year 1 of the crediting period (tCO2e/year) |
| PEy1 | = | Project emissions in year 1 of the crediting period (tCO2e/year) |
| UNCBEact/hist,y1 | = | Uncertainty of baseline emissions at the lower bound of the 95 per cent confidence interval relative to the central estimate of quantified unadjusted net baseline in year 1 of the crediting period (fraction) |
| y1 | = | Calendar year of the start date of the first crediting period |

- 10.3.2. Downward adjusted baseline emissions in subsequent years

- The downward adjusted baseline emissions in subsequent years are separately determined as applicable and the following applies

Where the baseline has been determined based on existing or actual/historical emissions adjusted downwards, the starting point for increasing the downward adjustment over time shall be the downward adjusted baseline values in the calendar year of the start date of the first crediting period, as determined in section 10.3.1.

Where the baseline has been determined based on the best available technology, the starting point for increasing the downward adjustment over time shall be the emissions corresponding to the best available technology in the calendar year of the start date of the first crediting period, as determined in section ‎10.3.1;

The annual increase in the downward adjustment shall be applied starting on 1st January of a calendar year. The first increase shall be applied in the calendar year following the calendar year of the start date of the first crediting period. A pro-rata approach may be used to apply this minimum value to periods other than a full calendar year.

The downward adjusted baseline emissions from the A6.4 activity are determined based on the equation below:

- BEadj,y=BEy-DAy1+BEy1×0.01×y-y1                       Equation (9)

- Where:

| BEadj,y | = | Downward adjusted baseline emission (tCO2/year) |
| --- | --- | --- |
| y | = | Calendar years of the crediting period after the calendar year of the start date of the crediting period |
| y1 | = | Calendar year of the start date of the crediting period |
| DAy1 | = | Initial downward adjustment (t CO2e/year) |
| BEy1 | = | Baseline emissions in the calendar year of the start date of the first crediting period (t CO2/year) |

Identification of the conservative BAU scenario

The BAU scenario reflects the most plausible and conservative set of processes, technologies, and regulatory requirements for ammonia production in the host country or relevant region. The conservative business-as-usual (BAU) scenario for A6.4 activities eligible under this mechanism methodology are:

- Table 2: BAU scenario for A6.4 activities eligible under the mechanism methodology

| - Component | - Baseline approach (see Table 1 above) | - BAU |
| --- | --- | --- |
| - Ammonia production project activity – A6.4 activities implemented as retrofitted projects | - Existing actual or historical emissions | - The continuation of the preactivity scenario, i.e., the production of conventional ammonia using the fossil-based inputs and energy sources. |
| - Ammonia production project activity – A6.4 activities implemented as greenfield project. | - Best available technologies that represent an economically feasible and environmentally sound course of action | Establishment of an economically viable ammonia production technology in the baseline geographical reference area, selected from technologies commissioned in the past three years that together contribute to the top-two-quartile of cumulative installed capacity, representing the commercially prevailing and widely adopted technologies consistent with existing and planned policies. |

- In determining the BAU scenario and quantifying the BAU emissions, activity participants shall identify and incorporate in the BAU:

  - Any policies that are active or scheduled to take effect within the crediting period, unless they refer to or formally integrate the mechanism as an instrument for implementation. All legal requirements shall be deemed to be enforced while recognizing that regulatory environments vary; and

  - Any specific national or sub-national targets16 for the sector or the type of activity, as long as these are supported by policy frameworks for implementation, but not general goals that are not specific to the sector or type of activity (e.g. national emissions target).

Calculation of the conservative BAU emissions

- BAU emissions for both scenarios of A6.4 activity under this methodology are determined as follows:

- A. A6.4 activities implemented as retrofitted projects — Baseline based on the existing actual or historical emissions

- BAUy   =PNH3,y *  SEhistorical                            Equation (10)

- Where:

| BAUy | = | Business-as-usual emissions in year y (tCO2e/year) |
| --- | --- | --- |

- B. A6.4 activities implemented as greenfield project — Baseline based on the BAT (if BAT represents conventional new-build in region)

- BAUy   =PNH3,y *  SENH3,BAT                                Equation (11)

- Where:

| SENH3,BAT | = | Average CO₂ emissions intensity of the ammonia production plants representing the economically viable technologies most adopted in the baseline geographical reference area, as determined using the top-two-quartile cumulative installed capacity approach. (t CO2e/tNH3) |
| --- | --- | --- |

- 10.4.1.1 Identification of economically viable technologies most widely implemented in the baseline geographical reference area using the top-two-quartile cumulative installed capacity approach

- 10.4.1.1.1 Step 1: Identification of the baseline geographical reference area

The baseline geographical reference area is the host country by default.

Activity participants may choose to limit the baseline geographical reference area to a narrower specific geographical area (such as municipality, province, states, etc.) within the host country if they can demonstrate and justify that there are essential distinctions between the identified specific geographical area and rest of the host country. For example, such distinctions may include certain benefits that make the A6.4 activity financially/economically more attractive (e.g., subsidies or other financial flows), different climatic conditions.

- 10.4.1.1.2. Step 2. Identify technologies and/or practices implemented in the baseline geographical reference area

Consider all ammonia production technologies/plants commissioned over the three years preceding the project start date in the relevant geographical area.

- 10.4.1.1.3. Step 3. Identification of the economically viable technologies

Out of the technologies and/or plants that are identified in Step 2 above, activity participants shall further identify those that are deemed economically viable (i.e., those that provide sufficient returns to cover investment, operations, and maintenance costs).

Technologies and/or practices are deemed economically viable if they:

- (a) Have been implemented under prevailing market conditions without significant subsidies by at least one facility within the applicable baseline geographical reference area without the revenues from carbon credits or other incentives; or

- (b) Are accessible off the shelf or via tendering within the applicable baseline geographical reference area, on terms that are economically viable when compared against a benchmark.

To identify the economic viability of a technology or practice under paragraph 73(b), activity participants shall conduct an investment analysis (using benchmark analysis approach) following the same requirements used for the additionality demonstration as per section 9.3.1 above and shall:

- (a) Collect relevant information directly from technology or practice suppliers; or

- (b) Demonstrate that it is not possible to collect relevant information directly from technology or practice suppliers. In this case, activity participants shall:

- (i) Where direct cost information from local suppliers is unavailable, cost and financial parameters may be derived from benchmarks, historical data from similar facilities, peer-reviewed literature, country-specific cost reports, or internationally recognized studies/standards. Clear justification shall be provided to demonstrate that such information reasonably reflects conditions within the applicable baseline geographical reference area. The DOE shall cross-verify this during validation and project registration.

- (ii) Provide proper justification that any such information can reasonably be applied in the baseline geographical reference area. The DOE shall cross-verify the same during the validation and project registration stage

- 10.4.1.1.4. Step 4. Rank economically viable technologies by emissions intensity

Rank all economically viable technologies/plants based on CO₂ emissions intensity, from lowest to highest.

Record the installed capacity of each technology for use in cumulative calculations in the baseline geographical area.

- 10.4.1.1.5. Step 5. Determine the top-two-quartile capacity threshold

Compute the total installed capacity of all economically viable technologies in the baseline area:

Sequentially select technologies from lowest to highest CO₂ intensity, adding their capacities cumulatively.

Stop when the cumulative installed capacity reaches 50% of the total installed capacity.

- Top-two-quartile threshold = 0.5 × Total number of plants implemented

- 10.4.1.1.6. Step 6. Select technologies to fill top 50%

Include technologies sequentially from Step 5 until cumulative capacity ≥ top-two-quartile threshold. These selected technologies represent the BAU “most widely adopted” and best available technology capacity for the area.

- 10.4.1.1.7. Step 7. Determine capacity-weighted average CO₂ emissions intensity of economically viable technologies, most widely implemented

Collect emissions intensity (EI) for all the selected technologies (tCO₂e/tNH₃)

- Capacity-weighted average emissions intensity:

- SENH3,BAT=  i(EIi×Capi)i(Capi)                                 Equation (12)

- Where:

| EIi | = | CO₂ emissions intensity of technology i (t CO₂e/tNH₃) |
| --- | --- | --- |
| Capi | = | Installed ammonia production capacity of technology i (t NH₃/year) |

Activity participants shall:

- (a) Collect relevant information directly from technology or practice suppliers; or

- (b) Demonstrate that it is not possible to collect relevant information directly from technology or practice suppliers. In this case, activity participants:

- (i) May determine the emissions intensity from similar facilities located outside of the baseline geographical reference area, or from peer reviewed studies and papers; and

- (ii) Shall provide proper justification that any such information can reasonably be applied in the baseline geographical reference area

- 10.4.1.2 The conservative BAU emissions shall be determined based on the stepwise process below:

- 10.4.1.2.1 Step 1. Determine the uncertainty at the lower bound of the uncertainty interval (UCNBAU, CP1,y)

The 95 per cent confidence interval for uncertainty shall be determined considering uncertainty in all parameters used to calculate BAU emissions during the first crediting period following the guidance from Volume 1, Chapter 3 of the IPCC (2019 Refinement) through the error propagation method or through the Monte Carlo simulation.

Uncertainties should be traceable for each and every parameter, with clear values and assumptions for uncertainty and any associated references for estimating uncertainties of parameters

- 10.4.1.2.2 Step 2. Determine the conservative business-as-usual baseline emissions and/or removals based on uncertainty (BEcons,UNC,y)

- Determine the conservative BAU emissions based on the uncertainty (BAUcons, UNC, y) as follows:

| BAUcons, UNC,y=BAUy×1-UNCBAU, CP1, y | Equation (13) |
| --- | --- |

Where:

| BAUcons, UNC,i,y | = | Conservative BAU baseline emissions based on the uncertainty in year y (tCO2e/year) |
| --- | --- | --- |
| BAUi,y | = | Most likely net BAU baseline emissions in year y (tCO2e/year), determined based on guidance from paragraph ‎66 above |
| UNCBAU, CP1, y | = | Uncertainty at the lower bound of the uncertainty interval relative to the central estimate of the ex-ante quantified most likely net BAU baseline emissions during the first crediting period year y (fraction) |
| y | = | Relevant year or period |

- 10.4.1.2.3 Step 3. Determine the minimum conservative value of the business-as-usual baseline (BAUcons,min,y)

| BAUcons, min,y=BAUy-BAUy- PEy ×0.1 | Equation (14) |
| --- | --- |

Where:

| BAUcons, min, y | = | Minimum conservative BAU baseline emissions in year y (tCO2e/year) |
| --- | --- | --- |
| PEy | = | Project emissions in year y (tCO2e/year). |
| y | = | Relevant year or period |

- 10.4.1.2.4 Step 4. Compare BAUcons,UNC,i,y and BAUcons,min,i,y

- The conservative BAU baseline emissions shall be the lowest between BAUcons,UNC,y and BAUcons,min,y, as follows:

| BAUcons,y=minBAUcons,min,y ; BAUcons,UNC,y | Equation (15) |
| --- | --- |

Where:

| BAUcons,y | = | Conservative BAU baseline emissions in year y (tCO2e/year) |
| --- | --- | --- |
| y | = | Relevant year or period |

- The BAU scenario shall be determined ex ante and described in the PDD at the start of the first crediting period for the same duration as the crediting period of the proposed A6.4 activity and shall be redetermined at each crediting period renewal.

- The quantification of the BAU emissions shall be determined:

  - Ex ante and in the PDD at the start of the first crediting period for the same duration as the crediting period of the proposed A6.4 activity and specified for each calendar year within the crediting period; and

  - Ex post for each calendar year within the crediting period.

Comparison of the downward adjusted baseline and the conservative business-as-usual baseline

Activity participants shall compare, in the PDD, the downward adjusted baseline (BEadj,y) determined in section 10.3.2 with the conservative BAU baseline (BAUcons,y) determined in section ‎10.4.1 above.

- ∆d= BAUcons,y- BEadj,y                                Equation (16)

- If, as a result of the comparison, the ex-ante BAUcons,y is lower than the ex-ante BEadj,y for any calendar year or cumulatively over the crediting period, activity participants shall revise the quantitative methods and factors to determine the downward adjustment in section ‎10.3 above to ensure that the downward adjusted baseline is lower than the conservative BAU baseline for each calendar year and cumulatively for the crediting period or use a discounted value of conservative BAU (BAUcons,y).

- Activity participants shall also compare in monitoring reports, for each individual calendar year during the crediting period, the ex-post calculated downward adjusted baseline for the year and the ex-post calculated conservative BAU baseline for the same year to confirm that the downward adjusted baseline is lower than the conservative BAU baseline. If it is not, then the conservative BAU baseline shall be used for that specific calendar year.

- Activity scenario

Calculation of project activity emissions

Steps for calculating activity emissions in low carbon/green ammonia production project activity

- Step 1: Identify the sources of activity emissions in the defined project boundary

- Activity emissions are calculated for the following key process stages:

- Hydrogen Production (Electrolysis); Desalination/Treatment of water and Pumping

- Air Separation for Nitrogen

- Ammonia Synthesis (Haber-Bosch Process)

- Step 2: Project activity emissions shall be calculated as follows:

PEy = PEelectricity,y+PEfuel,y+PEleaks,y               Equation (17)

Where:

| PEy | = | Project activity emissions in year y (tCO2) |
| --- | --- | --- |
| PEelectricity | = | Project activity emissions resulting from the use of electricity from non-renewable sources across the entire low carbon/green ammonia production process project activity in year y (tCO₂e) |
| PEfuel | = | Project activity emissions resulting from the consumption of fossil fuel across the entire low carbon/green ammonia production process project activity in year y (tCO₂e) |
| PEleaks, y | = | Project activity emissions resulting from the physical leaks of hydrogen and ammonia in ammonia production project activity in a year y (tCO₂e) |

- Step 3: Project activity emissions from electricity use

PEelectricity = EGelec,grid,y* EFelec,grid+ EGelec,RE,y​×EFelec,RE,y​         Equation (18)

Where:

| EGelec,grid,y | = | Amount of electricity consumed in the project activity from grid in a year y (MWh/year) |
| --- | --- | --- |
| EFelec,grid | = | Emission factor for grid electricity (tCO2/MWh) |
| EGelec,RE,y | = | Amount of electricity consumed from renewable energy sources in year y (MWh/year) |
| EFelec,RE,y | = | Emission factor for renewable electricity (tCO2/MWh) |

- If the renewable electricity is sourced from solar or wind power plants, its emission factor shall be considered zero. However, if it is sourced from hydro or pumped hydro storage, project participants shall account for associated CH₄ and CO₂ emissions from the reservoirs, using default values or project-specific estimates as applicable.

EFelec,RE,y= EFRes                                          Equation (19)

Where:

| EFRes | = | Default emission factor for emissions from reservoirs of hydro power plants/ PSP (kg CO2e/MWh) |
| --- | --- | --- |

- Step 3: Project activity emissions from fossil fuel use

PEfuel,y = Fconsumed,iy* EFi,y                                 Equation (20)

Where:

| Fconsumed,i,y | = | Quantity of fuel type i, consumed in the whole ammonia production project activity in a year y (tons/year) |
| --- | --- | --- |
| EFi,y | = | Emission factor for fuel type i consumed in the whole ammonia production project activity in a year y (tCO2/ton of fuel) |

- Step 4: Project activity emissions from physical leaks of Hydrogen and Ammonia

PEleaks,y = PEH2-leaks,y+ PENH3-leaks,y                       Equation (21)

Where:

| EH2-leaks, y | = | Project activity emissions due to physical leaks of hydrogen in the project activity in a year y (tCO2e) |
| --- | --- | --- |
| ENH3-leaks, y | = | Project activity emissions due to physical leaks of ammonia in the project activity in a year y (tCO2e) |

- Emissions due to physical leaks of hydrogen

- PEH2-leaks = QH2,green,y * PLH2 * GWPH2                                   Equation (22)

Where:

| QH2,green,y | = | Mass of pure hydrogen produced by the electrolyzer using renewable energy and consumed for low carbon/green ammonia production in year y (tH₂). |
| --- | --- | --- |
| PLH2,y | = | Physical leaks of green hydrogen across the hydrogen value chain expressed as a percentage of the total green hydrogen produced in a year y (%) |
| GWPH2 | = | Global warming potential of hydrogen (tCO2e/tH2) |

- Emissions due to physical leaks of nitrogen

- PENH3-leaks = PNH3,y * PLNH3 * fNH3 →NOx* GWPNO2               Equation (23)

Where:

| PNH3,y | = | Quantity of low carbon/green ammonia produced in PACM project activity in year y (tNH3) |
| --- | --- | --- |
| PLNH3,y | = | Physical leaks of low carbon/green ammonia in ammonia value chain as a percentage of the total production in a year y (%) |
| GWPNO2 | = | Global warming potential of nitrous oxide (tCO2e/tNH3) |
| fNH3→NOx | = | Fraction of volatilized ammonia (NH₃) that undergoes atmospheric oxidation to form nitrogen oxides (NOₓ) (%) |

- Note: Based on the some of the research, fraction of volatilized ammonia that undergoes atmospheric oxidation to from nitrogen oxides (NOx) is very minimal (i.e.: lesser than 1%). Hence this can be neglected if country specific oxidation factor is not available.

- Leakage

Identification of leakage emission sources

In accordance with the requirements of the Standard for Addressing Leakage in Mechanism Methodologies (A6.4-STAN-METH-005) (Version 1.0), this methodology identifies all potential sources of leakage relevant to low carbon/green ammonia production project activity as required under para. 12.

The following potential leakage sources are considered:

- Baseline equipment transfer (para. 12(a)).

- Competition for resource use (para. 12(b)).

- Diversion of existing production or outputs (para. 12(c)).

- Environmental GHG release (para. 12(d)).

Each of these leakage sources is assessed for relevance to low carbon/green ammonia production projects

Avoidance or minimization of leakage

Baseline equipment transfer (Applies to the brownfield project activities):

- Leakage may occur if conventional hydrogen production equipment replaced by the project is reused elsewhere. In accordance with paragraph 82(b) of the standard, the methodology requires either

- (a) proof of permanent decommissioning or scrapping of the baseline equipment, or

- (b) quantification of continued emissions if reused.

- Where the baseline equipment continues to operate, leakage is estimated based on operating hours, fuel use, output, and emission factors. If dismantled and repurposed in a disaggregated manner, no leakage is considered. Thus, leakage is avoided or minimized through decommissioning or quantification measures.

Competition for resource use:

- Leakage may occur if the project consumes resources that have limited availability and competing uses. To avoid leakage from competing electricity and water use, the methodology includes the following applicability conditions:

- At least 90% of project electricity must be supplied by newly built renewable power plants, thereby avoiding displacement of existing clean capacity.

- Project water use shall not exceed 5% of locally available renewable freshwater resources, ensuring no significant diversion from other users.

- Provided these conditions are met, leakage from resource competition is considered negligible and conservatively set to zero.

Diversion of existing production or outputs (applies only to retrofitted activities)

- For brownfield project activities, leakage from diversion of production is avoided by mandating that the project delivers the same output/service level as the baseline. The applicability conditions of the methodology specify that the output(s) or level(s) of service delivered by the project must remain identical to those in the pre-PACM project condition. As this requirement ensures there is no diversion of production or change in service level, calculation of leakage emissions related to such changes is not required. Therefore, the risk of leakage related to production diversion is effectively mitigated and can be excluded from the leakage assessment for simplification purposes. For brownfield project activities, this methodology requires that the project delivers the same output(s) or level(s) of service as the pre-PACM project condition. Under this applicability condition, leakage from diversion of production does not occur and is excluded from the leakage assessment.

Indirect environmental emissions

- For greenfield project activities, if the land was used for agriculture or if any land-use change has occurred prior to the implementation of the PACM project activity, the associated emissions shall be accounted for and calculated in accordance with the requirements of this methodology.

Addressing leakage emissions

Where leakage is identified as significant, it shall  be quantified using conservative assumptions and, where available, default emission factors or project-specific data.

- The total leakage emissions (LEtotal) for a given year shall be calculated as:

LEy​=LEequip,y​+LEres,y​+LEdiv,y​+LEenv,                                                 Equation (24)

Where:

| LEy | = | Total leakage emission in a year y (tCO2) |
| --- | --- | --- |
| LEequip,y | = | Leakage from transferred baseline equipment in a year y (tCO2) |
| LEres,y | = | Leakage from resource competition in a year (tCO2) |
| LEdiv,y | = | Leakage from diversion of production in a year (tCO2) |
| LEenv,y | = | Leakage from indirect environmental emissions in a year (tCO2) |

Leakage from transferred baseline equipment (LEequip,y): Leakage from transferred baseline equipment (LEequip,y) applies only to retrofitted project activities where the baseline conventional ammonia production plant uses hydrogen-producing equipment other than electrolyzers, and such equipment is not disposed of or scrapped but continues to be used for the same purpose after the implementation of the A6.4 activity.

- LEequip,y​=Qoutput,y​× EFHydrogen-equip​                            Equation (25)

Where:

| Qoutput,y | = | Amount Hydrogen produced from baseline transferred equipment in a year y (tH2/year) |
| --- | --- | --- |
| EFHydrogen-equip | = | Emission factor of the baseline conventional hydrogen production equipment per ton of the hydrogen output (tCO₂e/tonH2) |

Leakage from resource competition (LEres,y):  This methodology requires that at least 90% of electricity is supplied from renewable sources and water consumption does not exceed 5% of local availability. Under this applicability condition:

- LEres,y​ = 0                                              Equation (26)

Leakage from diversion of production (LEdiv,y): For brownfield project activities, this methodology requires that the project delivers the same output(s) or service level as the pre-PACM project condition. Under this applicability condition:

- LEdiv,y = 0			                    Equation (27)

Leakage from indirect environmental emissions (LEenv,y):  For greenfield project activities, if land was previously used for agriculture or if any land-use change occurs prior to the PACM project, associated emissions shall be quantified using IPCC default factors or host country–approved data:

LEenv,y​=g∑​(Qg,y​×GWPg​)                                           Equation (28)

Where:

| Qg,y | = | Quantity of GHG type g released due to land-use change in year y (tons) |
| --- | --- | --- |
| GWPg | = | Global Warming Potential of GHG type g |

If the sum of leakage sources results in a net decrease in emissions (positive leakage), the value of leakage shall be set equal to zero, in accordance with para. 10 of the Leakage Standard.

- Emission reductions

For each year of the crediting period, emission reductions are calculated by applying the baseline, activity, and leakage formulations already defined, using monitored annual low carbon/green ammonia production data and verified emission factors.

For each year y of the crediting period, emission reductions are calculated as

- ERy = BEadj,y – PEy - LEy                                         Equation (29)

Where:

| ERy | = | Emission Reduction in year y (tCO2e) |
| --- | --- | --- |

- Data and parameters not monitored

Data / Parameter table 1.

| Data/parameter | SEBAT,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Specific emissions of the best available ammonia production technology |  |  |  |
| Data unit | (tCO2e/tNH3) |  |  |  |
| Equations referred | Equation 1 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | Determined based on procedure to identify the baseline scenario for ammonia production contained in section 10.2.2 above |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | As per the procedure contained in section 10.2.2 above |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 2.

| Data/parameter | ECO2,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Actual CO2 emissions from historical ammonia production in year y |  |  |  |
| Data unit | tCO₂ |  |  |  |
| Equations referred | Equation 4 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | As per the procedure contained in para 50 above. |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 3.

| Data/parameter | PNH3,historical,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Annual historical ammonia production from the conventional ammonia production facility during the pre-A6.4 project period in year y |  |  |  |
| Data unit | tNH₃ |  |  |  |
| Equations referred | Equation 4 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | As per the procedure contained in para 50 above. |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 4.

| Data/parameter | Capi |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Installed ammonia production capacity of technology i |  |  |  |
| Data unit | t NH₃/year |  |  |  |
| Equations referred | Equation 12 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | Determined based on procedure to identify the baseline scenario for ammonia production contained in section 10.4.1 above |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | As per the procedure contained in section 10.4.1 above |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |
| Additional comments | NA |  |  |  |

Data / Parameter table 5.

| Data/parameter | EIi |  |  |  |
| --- | --- | --- | --- | --- |
| Description | CO₂ emissions intensity of technology i |  |  |  |
| Data unit | tCO2e/tNH3 |  |  |  |
| Equations referred | Equation 12 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | Determined based on procedure to identify the baseline scenario for ammonia production contained in section 10.4.1 above |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | As per the procedure contained in section 10.4.1 above |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |
| Additional comments | NA |  |  |  |

Data / Parameter table 6.

| Data/parameter | GWPH2 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global warming potential of Hydrogen |  |  |  |
| Data unit | tCO2/tonH2 |  |  |  |
| Equations referred | Equation 22 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 5.8 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC AR4 WG1 as under chapter 2.10.3.6 |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 7.

| Data/parameter | GWPNO2 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global warming potential of Nitrous Oxide |  |  |  |
| Data unit | tCO2/tonNH3 |  |  |  |
| Equations referred | Equation 23 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 273 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Sixth Assessment Report, 2020 (AR6) |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 8.

| Data/parameter | fNH3→NOx |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Fraction of volatilized ammonia (NH₃) that undergoes atmospheric oxidation to form nitrogen oxides (NOₓ) (%) |  |  |  |
| Data unit | % |  |  |  |
| Equations referred | Equation 23 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Values shall be obtained from recognized peer-reviewed literature, IPCC Guidelines (2006, Vol. 4, Ch. 11, Agriculture), or country-specific defaults where available. |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | - |  |  |  |

Data / Parameter table 9.

| Data/parameter | EFRes |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Default emission factor for emissions from reservoirs of hydro power plants/ PSP |  |  |  |
| Data unit | kgCO2e/MWh |  |  |  |
| Equations referred | Equation 19 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 90 kgCO2e/MWh |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Decision at CDM EB 23 |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments |  |  |  |  |

Data / Parameter table 10.

| Data/parameter | EFHydrogen-equip |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor of the baseline conventional hydrogen production equipment per ton of the hydrogen output |  |  |  |
| Data unit | tCO₂e/tonH2 |  |  |  |
| Equations referred | Equation 25 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | Determined based on the emissions intensity of the baseline hydrogen production equipment (e.g., SMR, coal gasification, or other conventional units) using plant-specific documentation, as applicable |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Use credible and authoritative sources (e.g., manufacturer data, or documented plant performance reports) to determine the emission factor corresponding to the specific type of hydrogen production equipment used in the baseline scenario. |  |  |  |
| Treatment of uncertainty | Not applicable (NA). |  |  |  |
| Additional comments | This parameter is applicable only for brownfield project activities where the baseline hydrogen production equipment continues to operate after the implementation of the A6.4 activity. |  |  |  |

Data / Parameter table 11.

| Data/parameter | GWPg​ |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global Warming Potential of GHG type g |  |  |  |
| Data unit | tCO2/tonGHGs |  |  |  |
| Equations referred | Equation 28 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | Greenhouse Gas (GHGg); Chemical Formula; GWP (100-year). Carbon dioxide; CO₂; 1. Methane; CH₄; 28. Nitrous oxide; N₂O; 265. |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Fifth Assessment Report of the IPCC |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | - |  |  |  |

- Data and parameters monitored

Data / Parameter table 1.

| Data/parameter | QNH3,total,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Total quantity of ammonia produced and verifiably exported by project activity during the year y |  |  |  |  |
| Data unit | tNH3 |  |  |  |  |
| Equations referred | Equation 2 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously by flow meters/weighbridge and recorded in daily logs; aggregated monthly and reported annually. |  |  |  |
| Measurement methods and procedures | Low carbon/green ammonia production and export will be determined based on continuous readings from calibrated flow meters at the ammonia synthesis outlet or weighbridge records (for bulk transfers). These measurements will be cross-checked with production records, dispatch notes, and stock balances. Where only a portion of the total ammonia output qualifies as green, the fraction fgreen,y derived from the share of renewable hydrogen and renewable nitrogen inputs shall be applied to the total measured ammonia production to determine the quantity of low carbon/green ammonia that is eligible for crediting. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Calibrated flow meter (continuous measurement) / weighbridge (batch-wise transfer), verified against production logs. |  |  |
|  | Accuracy class |  | Minimum ±1% of measured value. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | At project site, downstream of ammonia synthesis unit / dispatch point. |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data on a monthly basis. - Annual third-party verification of records and calibration certificates. - Electronic data backed up, and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | Apply uncertainties based on the IPCC (2019 Refinement) |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 2.

| Data/parameter | ECelec,grid,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Amount of electricity consumed in the project activity from grid in a year y |  |  |  |  |
| Data unit | MWh/year |  |  |  |  |
| Equations referred | Equation 18 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Electricity import from the grid shall be measured continuously using calibrated energy meters. Data are automatically logged, aggregated monthly, and reported annually. |  |  |  |
| Measurement methods and procedures | Electricity drawn from the grid is measured using utility-grade energy meters installed at the grid interconnection point. The meters record all electricity consumed for hydrogen production, nitrogen production, ammonia synthesis, and auxiliary loads. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Utility-grade energy meters integrated with SAP/SCADA for real-time data capture and reporting |  |  |
|  | Accuracy class |  | Class 0.2S or Class 0.5 as per national metering standards |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier 3. Responsible entity: Accredited third-party calibration agency / internal QA team |  |  |
|  | Location |  | Electricity metering point at the grid interconnection boundary of the project. |  |  |
| QA/QC procedures | - Routine calibration and maintenance as per SOP - Monthly cross-verification with utility bills - Data archived digitally via SCADA/SAP and retained for auditing |  |  |  |  |
| Treatment of uncertainty | Uncertainty minimized through the use of calibrated instruments. Missing or inconsistent data shall be treated conservatively to avoid overestimation of emissions. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 3.

| Data/parameter | ECelec,RE,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Amount of electricity consumed from renewable energy sources in year y (MWh/year) |  |  |  |  |
| Data unit | MWh/year |  |  |  |  |
| Equations referred | Equation 18 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Electricity supplied from renewable sources shall be continuously measured using calibrated utility-grade meters. Data are automatically recorded, aggregated monthly, and reported annually. |  |  |  |
| Measurement methods and procedures | Renewable electricity - whether generated onsite or wheeled from an offsite RE plant - is measured at: - the RE generator export point, and/or - the project site import point, using utility-grade bidirectional energy meters. These meters record renewable electricity supplied for hydrogen production, nitrogen separation, ammonia synthesis, and all auxiliary project operations. Cross-checks are performed using SLDC schedules, RE generator invoices, and SCADA logs. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Utility-grade bidirectional energy meters integrated with SCADA/SAP |  |  |
|  | Accuracy class |  | Class 0.2S or Class 0.5 as per national metrology standards |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier 3. Responsible entity: Accredited third-party calibration agency / internal QA team |  |  |
|  | Location |  | RE generator export point; Project site import point (for wheeled power) |  |  |
| QA/QC procedures | - Monthly reconciliation of RE generator invoices and SLDC schedules - Calibration logs and utility billing cross-checks - Real-time SCADA-based data logging and archival for verification |  |  |  |  |
| Treatment of uncertainty | Uncertainty minimised through use of high-accuracy, calibrated meters. Any missing or inconsistent data shall be treated conservatively to avoid overestimation of emission reductions. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 4.

| Data/parameter | EFelec,grid |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Emission factor for grid electricity |  |  |  |  |
| Data unit | tCO2/MWh |  |  |  |  |
| Equations referred | Equation 18 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Updated annually based on the most recent official grid emission factor published by the national authority, designated entity, or the applicable A6.4 emission factor tool (once available). |  |  |  |
| Measurement methods and procedures | Refer to the A6.4-MEP010-A01 - Emissions from electricity generation and/or consumption |  |  |  |  |
| Entity/person responsible for the measurement | NA |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | NA |  |  |
|  | Accuracy class |  | NA |  |  |
|  | Calibration requirements |  | NA |  |  |
|  | Location |  | NA |  |  |
| QA/QC procedures | - Verify that the grid emission factor is taken from the latest official and quality-assured source. - Maintain copies of source documents, official publications, and calculation sheets. |  |  |  |  |
| Treatment of uncertainty | NA |  |  |  |  |
| Additional comment | This section will be updated once the relevant A6.4 tool for grid electricity emission factor calculation becomes available. |  |  |  |  |

Data / Parameter table 5.

| Data/parameter | Fconsumed,iy |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Quantity of fuel type i, consumed in the whole ammonia production project activity in a year y |  |  |  |  |
| Data unit | tons/year |  |  |  |  |
| Equations referred | Equation 20 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Fuel consumption shall be determined from inventory balance records, supplier invoices, delivery notes, and stock registers. Data are updated monthly and cross-checked annually. |  |  |  |
| Measurement methods and procedures | Fuel consumption during the monitoring period shall be determined using the following steps: - Record opening fuel stock at the beginning of year y. - Add fuel purchased/received during year y (from invoices and delivery notes). - Record closing stock at the end of year y. - Calculate net fuel consumed as: Fconsumed,i,y = Opening stock (tons) + Purchased (tons) – Closing stock (tons) For liquid fuels, flow meter readings at the receiving and consumption points shall be used and cross-verified with inventory records. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Weighbridge (for solid fuels) / calibrated flow meter (for liquid fuels), cross-verified with purchase invoices. |  |  |
|  | Accuracy class |  | NA. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | Fuel storage and receiving areas. |  |  |
| QA/QC procedures | - Cross-check fuel consumption data with purchase records, supplier invoices, and stock registers. - Reconciliation of measured data with invoices to avoid errors. - Periodic verification of measurement records through internal audits. - Backup of electronic data and maintenance of hard copies of invoices and stock records. |  |  |  |  |
| Treatment of uncertainty | - Use of calibrated instruments to minimize measurement uncertainty. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 6.

| Data/parameter | EFi,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Emission factor for fuel type i consumed in the whole ammonia production project activity in a year y |  |  |  |  |
| Data unit | tCO2/ton of fuel |  |  |  |  |
| Equations referred | Equation 20 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Updated annually or whenever IPCC/national default values are revised. |  |  |  |
| Measurement methods and procedures | The emission factor shall be sourced from: - IPCC 2006/2019 Guidelines, - National GHG inventory database, or - Peer-reviewed national emission factors (if available). |  |  |  |  |
| Entity/person responsible for the measurement | Activity participant |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | NA |  |  |
|  | Accuracy class |  | NA. |  |  |
|  | Calibration requirements |  | NA |  |  |
|  | Location |  | NA |  |  |
| QA/QC procedures | - Verify the EF source reference (IPCC or national document). - Ensure the selected EF aligns with the correct fuel type and carbon content. - Maintain documented evidence of source, version, and year. |  |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by using IPCC or nationally approved default factors with published uncertainty ranges. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 7.

| Data/parameter | PLH2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Physical leaks of green hydrogen across the hydrogen value chain expressed as a percentage of the total green hydrogen produced in a year y |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 22 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring using fixed hydrogen gas detectors and periodic LDAR inspections. Data aggregated monthly and reported annually. |  |  |  |
| Measurement methods and procedures | Hydrogen leakage shall be determined using: - continuous readings from fixed green hydrogen gas leak detectors installed in the production, compression, storage, and transfer systems. - LDAR (Leak Detection and Repair) surveys using portable hydrogen detectors. - cross-checking with mass-balance records: PLH2,y=Measured hydrogen loss (t)Total green hydrogen produced (t)×100 |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Hydrogen gas detectors (electrochemical, TCD, catalytic bead, infrared) integrated with SCADA/SAP; portable hydrogen analyzers for LDAR. |  |  |
|  | Accuracy class |  | Sensor-specific, typically ppm-level or %LEL accuracy. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | Hydrogen production, compression, pipeline corridors, buffer storage, and transfer/loading areas. |  |  |
| QA/QC procedures | - Review SCADA leakage alerts, LDAR survey reports, calibration certificates. - Monthly reconciliation of production vs. dispatch/consumption records to validate mass balance. - Internal audits of leak logs and maintenance records. - Digital logs backed up; hard-copy documents archived securely |  |  |  |  |
| Treatment of uncertainty | Minimized through calibrated instruments; missing data treated conservatively. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 8.

| Data/parameter | QH2,green,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Mass of pure hydrogen produced by the electrolyzer using renewable energy and consumed for low carbon/green ammonia production in year y |  |  |  |  |
| Data unit | tH₂ |  |  |  |  |
| Equations referred | Equation 22 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously, aggregated monthly, and reported annually. |  |  |  |
| Measurement methods and procedures | Measure total hydrogen utilized in the ammonia production project activity in year y (QH₂,total,y). |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Flow meters, weighbridges, production logs, SCADA/SAP data |  |  |
|  | Accuracy class |  | Instrument-specific; typical industrial flow meters or production meters ±1–2% accuracy |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards 2. Calibration frequency: At least once per year or as recommended 3. Responsible entity: Internal QA team or accredited third-party calibration agency. |
|  | Location |  | Hydrogen production units, ammonia synthesis feed lines, buffer storage. |  |  |
| QA/QC procedures | - Monthly reconciliation of production vs. consumption records - Internal audits and cross-checks - Backup of digital logs and archiving of hard-copy records |  |  |  |  |
| Treatment of uncertainty | NA |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 9.

| Data/parameter | PLNH3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Physical leaks of ammonia in ammonia value chain as a percentage of the total production |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 23 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring using fixed ammonia gas detectors and periodic LDAR inspections. Data aggregated monthly and reported annually |  |  |  |
| Measurement methods and procedures | Ammonia leakage shall be determined based on: - continuous readings from fixed ammonia gas detection sensors installed in production, transfer, and storage areas. - results from periodic LDAR surveys using portable gas detectors. - mass-balance verification between total measured production and dispatch/consumption records. Leakage percentage is calculated as: PLNH3,y=Measured ammonia loss (t)Total ammonia produced (t)×100 |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Gas leak detection sensors (e.g., ammonia detectors) are integrated with SAP or SCADA systems for real-time monitoring and alerts |  |  |
|  | Accuracy class |  | Sensor-specific; typically, qualitative or ppm-level accuracy |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | Ammonia production and storage areas |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data monthly. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | Uncertainty minimized through continuous monitoring with calibrated sensors. Any missing, inconsistent, or alarm-triggered data shall be treated conservatively. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 10.

| Data/parameter | Qoutput,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Amount Hydrogen produced from baseline transferred equipment in a year y |  |  |  |  |
| Data unit | tH2 |  |  |  |  |
| Equations referred | Equation 25 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously, aggregated monthly, and reported annually. |  |  |  |
| Measurement methods and procedures | Measure total hydrogen produced using the baseline equipment transferred in year y |  |  |  |  |
| Entity/person responsible for the measurement | Project developer |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | NA |  |  |
|  | Accuracy class |  | NA |  |  |
|  | Calibration requirements |  | NA |  |  |
|  | Location |  | Project site (land area affected before construction or project implementation). |  |  |
| QA/QC procedures | NA |  |  |  |  |
| Treatment of uncertainty | NA |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Data / Parameter table 11.

| Data/parameter | Qg,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Quantity of GHGs released due to land-use change in year y |  |  |  |  |
| Data unit | tGHGs |  |  |  |  |
| Equations referred | Equation 28 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | One-time estimation prior to project commissioning based on land clearance and soil disturbance. |  |  |  |
| Measurement methods and procedures | LUC emissions shall be estimated using: - IPCC 2006 Guidelines (Volume 4, AFOLU), - national forestry/soil carbon datasets (if available), - on-site assessment of vegetation cover, biomass density, soil type, and area cleared. |  |  |  |  |
| Entity/person responsible for the measurement | Project developer / environmental consultant |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | NA |  |  |
|  | Accuracy class |  | NA |  |  |
|  | Calibration requirements |  | NA |  |  |
|  | Location |  | Project site (land area affected before construction or project implementation). |  |  |
| QA/QC procedures | - Cross-check land-use classification using satellite imagery, GIS maps, or official land records. - Use the most conservative (highest reasonable) carbon stock values in accordance with IPCC guidelines. - Third-party validation of calculations and data sources. - Maintain all GIS shapefiles, imagery, calculation sheets, and supporting documents for verification. |  |  |  |  |
| Treatment of uncertainty | NA |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Frequency of submission of monitoring reports

This methodology does not involve activities that result in removals or emission reductions with a risk of reversal. Therefore, the provisions related to the maximum permissible interval between the start date of the crediting period and submission of the first monitoring report, as well as the interval.

- Methodologies principles

Encouraging ambition over time

This Mechanism methodology shall apply increasingly ambitious baselines to encourage ambition of activities over time, while considering host Party circumstances. and should facilitate the deployment of low carbon solutions and technologies.

Mechanism methodologies should facilitate the deployment of technologies or measures that are not widely used or available in specific locations, to facilitate knowledge transfer and encourage deployment of technologies or measures that reduce the cost of decarbonization and unlock investment in low-carbon solutions.

Mechanism methodologies should contain provisions that facilitate the inclusion of progressively more efficient and less greenhouse gas (GHG)-intensive technologies, replicable and scalable mitigation activities, expanding the user base, broader geographic coverage, and greater penetration of low-carbon solutions after initial deployment

In all the three approaches of baseline scenario determination, the ambition of the host party is clearly embedded, Over and above the methodology, makes provision for most ambitious approach amongst these. The difference with BAU and locking-in emissions are quantified.

Contributing to the equitable sharing of mitigation benefits between participating Parties

In this methodology, the most ambitious approach among all baseline scenarios ensures that emission reductions exceed BAU, directly contributing to the host Party’s NDCs and beyond. The low carbon/green ammonia project lifetime is ~25 years, double the crediting period, ensuring that a significant portion of emission reductions (~70%) benefits the host Party beyond the crediting period. Provisions of the methodology incorporate conditions specified by the Designated National Authority (DNA) of the host Party to align with national priorities. Mitigation benefits to the host Party are estimated using conservative assumptions, including adjustments for exogenous factors and crediting period alignment, and are explicitly linked to sustainable development objectives through the application of the SD Tool in project design and implementation. This approach ensures equitable sharing of mitigation benefits between participating Parties while tangibly supporting the host Party’s sustainable development goals.

Implementation and Verification:

- Credit period and project lifetime: The crediting period is clearly defined and shorter than the project lifetime, ensuring continued benefits to the host Party beyond the crediting period.

- DNA provisions: All conditions specified by the host Party’s DNA are incorporated during project design and implementation.

- Sustainable development linkage: Mitigation benefits are tied to SD objectives using the SD Tool; impacts are monitored and reported in accordance with the methodology.

- Ambitious estimation of benefits: Emission reductions and associated benefits to the host Party are calculated using conservative assumptions and adjustments for exogenous factors.

- Review and validation: Equitable sharing provisions and SD alignment are verified during validation and each crediting period renewal to ensure compliance and transparency.

The Designated Operational Entity (DOE) shall verify the equitable sharing provisions, estimation of mitigation benefits, and SD alignment during project validation and verification, as applicable.

Encouraging broad participation

This methodology is designed to encourage broad participation by being accurate, clear, and straightforward, ensuring that a wide range of project participants and host Parties - including those with limited scientific infrastructure or financial capacity - can implement it. It explicitly considers the context on the ground in host Parties, particularly in least developed countries and small island developing States, allowing flexibility to accommodate local conditions. The methodology recognizes and incorporates the knowledge and practices of local communities and Indigenous Peoples where relevant. To address potential data gaps, it permits the use of multiple data sources, conservative default values, and benchmarked data from comparable regions as applicable. Furthermore, the methodology is written in simple, inclusive, gender-sensitive language to ensure accessibility to a wide range of stakeholders, including local communities and Indigenous Peoples, thereby promoting equitable participation in the implementation of A6.4 activities

Attributability of emission reductions or net removals to the A6.4 activity

Attributability of emission reductions to the A6.4 activity is ensured as described in paragraph 73 (‘Identification of the conservative BAU scenario’), which accounts for exogenous factors and applies conservative adjustments in line with Section 5 of Appendix 1 of the Baseline Standard.

Potential perverse incentives

This methodology identifies the potential for a perverse incentive whereby activity participants may increase the production of ammonia to maximize the volume of emission reductions credited under the PACM mechanism.

Increased output could, in principle, inflate the calculated emission reductions if the mechanism rewards additional production that would not have occurred in the absence of the incentive.

However, this methodology ensures that such perverse incentives are avoided, through the following provisions:

- The installed production capacity of the ammonia plant is fixed at the time of validation and registration under the PACM mechanism. Any increase in capacity or significant change in output requires a design change request and revalidation, in accordance with the applicable A6.4 rules and procedures.

- 1. Only ammonia that is produced entirely from renewable hydrogen and nitrogen sources is eligible under the methodology.

- 2. Any increased production must be justified by actual demand and must occur within the constraints of available renewable inputs (e.g., electrolyser capacity, renewable power supply).

- 3. All input and output data are subject to third-party monitoring and verification to ensure that only eligible volumes are credited.

Through these design features, the methodology ensures that emission reductions cannot be artificially inflated and that output levels reflect legitimate activity expansion, not gaming of the mechanism.

Rebound effects

The issue of rebound effects is not relevant in the context of this methodology, which applies to the production of low carbon/green ammonia from renewable energy sources. While rebound effects are typically a concern where increased efficiency or reduced costs could lead to an increase in demand (e.g., lower electricity bills leading to higher appliance usage), the following considerations apply to low carbon/green ammonia project activity:

- 1. No impact on market prices:

- Low carbon/green ammonia production is especially in its early stages of market development, generally has higher production costs than fossil-based ammonia. As such, its implementation is not expected to reduce ammonia prices in the short to medium term. This significantly limits any potential for increased consumption due to price effects.

- 2. Policy and regulatory alignment:

- The scaling up of low carbon/green ammonia is often driven by climate policies, mandates, and premium markets (e.g., shipping fuel, green hydrogen mandates, or export requirements), rather than price incentives for farmers or industrial consumers.

- 3. Fixed capacity and output constraints:

- The methodology requires that eligible projects define installed capacity and operate under permits and operational constraints. Any increase in demand would not automatically lead to higher production unless new capacity is added, which would require a separate baseline scenario and methodological applicability check.

- Therefore, rebound effects do not apply or are negligible for this methodology.

Process description and flowcharts

1. Conventional Ammonia Production Process

2. Conventional Hydrogen Production Process

The CO2 emissions per kg of hydrogen produced can vary greatly depending on the method of production used.

    - Steam Methane Reforming (SMR) is the most common method of hydrogen production, and it typically results in CO2 emissions of around 8-10 kg CO2 per kg of hydrogen produced. This is because the process involves reacting methane (natural gas) with steam to produce hydrogen and CO2. The emissions from this process are directly proportional to the carbon content of the natural gas used.

    - Coal Gasification is another method of hydrogen production, it typically results in CO2 emissions of around 14-15 kg CO2 per kg of hydrogen produced. This is because the process involves reacting coal with steam to produce hydrogen and CO2. This method is highly polluting and energy intensive.

    - Steam reforming of natural gas offers an efficient, economical, and widely used process for hydrogen production, and provides near- and mid-term energy security and environmental benefits. The efficiency of the steam reforming process is about 65% to 75%, among the highest of current commercially available production methods. Natural gas is a convenient, easy to handle, hydrogen feedstock with a high hydrogen-to-carbon ratio. It is also widely available from sources in most of the countries across the world.

    - During the production of hydrogen, CO2 is also produced. The SMR process in centralized plants emits more than twice the CO2 than hydrogen produced.

    - Carbon Captue: To avoid emission of CO2 into the atmosphere, CO2 can be concentrated, captured, and sequestered; sequestration concepts and technologies are relatively new and there is no long-term test evidence to prove that these technologies will be successful. Sequestration in oceans is controversial because of the possible adverse impact on the aquatic environment by the reduction of ocean water pH.

    - Biomass Gasification is a method of hydrogen production that typically results in CO2 emissions of around 2-3 kg CO2 per kg of hydrogen produced. This is because the process involves reacting biomass (such as wood, agricultural waste, or sewage) with steam to produce hydrogen and CO2. The emissions from this process are considered to be low, as the CO2 produced is considered to be "carbon neutral" since it comes from a renewable source.

    - Electrolysis of water is a method of hydrogen production that typically results in CO2 emissions of around 1-2 kg CO2 per kg of hydrogen produced. The emissions from this process are directly proportional to the carbon content of the electricity used.

    - Photolysis is a method of hydrogen production that typically results in CO2 emissions of around 0 kg CO2 per kg of hydrogen produced. This is because the process involves using solar energy to split water into hydrogen and oxygen and does not produce any CO2 emissions.

    - Overall, the “cleanest” hydrogen production methods in terms of CO2 emissions per kg of hydrogen are electrolysis of water using renewable energy or photolysis.

    - The shift to a low-carbon, hydrogen-based economy is prompting the energy industry to explore more cost-effective and sustainable technologies, including methane pyrolysis (turquoise hydrogen), which offers significantly lower carbon emissions than steam methane reforming (SMR) and provides a more economical alternative to existing electrolysis-based green hydrogen production.

    - Methane pyrolysis's cost-effectiveness can be further enhanced through the effective utilization of the solid carbon byproduct, which none of the other competing technologies produce.

    - The solid carbon produced has potential applications across diverse sectors, such as electronics, energy storage systems, tire production, agricultural additives, and construction materials. Currently, emerging companies are at the forefront of methane pyrolysis research, development, and commercialization. Research encompasses thermal, thermocatalytic, and plasma decomposition methods for methane cracking, with each method offering unique advantages.

    - This study opens by offering a comparative analysis of methane pyrolysis with conventional hydrogen production technologies (SMR and water electrolysis). It covers multiple aspects of hydrogen production through methane pyrolysis, providing an overview of the thermal, thermocatalytic, and plasma pyrolysis processes.

The study evaluates each method's strengths and challenges and highlights the pioneering companies in each segment. In addition, it offers insight into the technology's driving forces and challenges and provides a techno-economic analysis of the various processes associated with methane pyrolysis. It also covers the patent landscape and offers a comprehensive analysis of the growth opportunities projected to play a pivotal role in driving the adoption of methane pyrolysis technology.

3. Conventional Nitrogen Production Process

The PSA process: The steps in the PSA process of nitrogen generation involve passing compressed air through a tower containing an adsorbent material such as a carbon molecular sieve that adsorbs oxygen in preference to nitrogen. The oxygen in the compressed air is removed by the adsorbent material, leaving behind nitrogen. When the adsorbent material is saturated, the oxygen in it is desorbed. In practice, two towers with adsorbent material are used alternately so that the production of nitrogen is continuous: Step 1 Nitrogen production using a pressure swing adsorption process.

Air separation plants extract gases from the air by distillation, primarily producing oxygen and nitrogen. Some facilities also recover argon, carbon dioxide, helium, neon, krypton, and xenon. These plants can be designed to produce either liquid or gaseous products and customized to extract specific gases while discarding others.

4. Emissions from ASUs

Air separation plants generate minimal emissions, mainly consisting of air gases and impurities returned to the atmosphere. However, accidental spills or hazardous releases must be reported. Potential emissions include:

- Air gases: Separated components like nitrogen and oxygen.

- Impurities: Naturally occurring substances in the air that are released back.

- Combustion emissions: Some backup systems use fired heaters, which may produce minor emissions.

Air Separation Methods

- Fractional distillation: The most common method.

- Cryogenic air separation units (ASUs): Used for producing nitrogen, oxygen, and sometimes argon.

- Membrane separation: Extracts a single component from the air.

- Pressure swing adsorption (PSA): Separates specific gases.

- Vacuum pressure swing adsorption (VPSA): Another method for isolating gases.

Overall, air separation processes produce very low emissions. Some cryogenic plants use small amounts of natural gas or electricity to heat pre-purifiers, but emissions remain minimal.

- - - - -

Document information

| Version | Date | Description |
| --- | --- | --- |
| 0.2.0 | 19 August 2025 | Revision to incorporate new sections and sub-sections in line with current standards; provide completion instructions, realign their sequence, and allow inclusion of explanatory notes. |
| 01.0 | 18 December 2024 | Initial publication of form template. |
| Decision Class: Regulatory Document Type: Form Business Function: Methodology Keywords: A6.4 mechanism, developing methodologies and tools |  |  |

- Format for footnotes.

In an electricity grid system where multiple injections by electricity generators and withdrawals of electricity by consumers occur, the grid authorities have to adopt systems of accounting for balancing/stability of the grid. In case of the market-based systems, the traceability is managed based on the scheduling, metering, and settlement framework. For example, in the context of EU, it has introduced the Electricity Balancing Guideline (EB-GL) (Regulation (EU) 2017/2195) as part of its internal energy market design. Whereas, in the context of India, this is implemented through the Deviation Settlement Mechanism (DSM), which records scheduled and actual injections and withdrawals at each grid connection point and supports accounting and verification of electricity delivery and consumption. Volumetric and temporal traceability of renewable electricity consumption for the green ammonia project can be demonstrated through time-block-wise metering and scheduling, with DSM data used for settlement and verification, enabling accounting-based matching of renewable electricity generation and project electricity consumption.

When full or partial impact of the activity is covered under mandatory domestic mitigation scheme and counted towards the achievement of targets and obligations under mandatory domestic mitigation scheme, the relevant share of the impact shall be deducted by the activity participants from the amount requested for issuance.

- For BAT determination, only the primary hydrogen production pathways (para 42) are considered baseline technologies. CCS and hybrid approaches are project measures applied alongside baseline technologies. Hydrogen production dominates GHG emissions which constitutes approximately 90-95% of the emission, hence the only hydrogen production pathways are considered for the BAT

In situations where a facility undergoes retrofit activities and normal operations are suspended/shutdown for an extended period, the baseline shall rely on the most recent operating history immediately prior to the start of the retrofit works. Only the operating years before the retrofit period and before operations resume shall be considered.

- The extent to which the policy frameworks in place are sufficient to enable the achievement of the policies/targets may be considered in determining their relevance for the BAU scenario.