|  | PROPOSED New baseline and Monitoring methodology oR methodological tool FORM FOR EMISSION REDUCTIONS ACTIVITIES (Version 02.0) |  |
| --- | --- | --- |
| Information to be completed by the secretariat and Methodologies Expert Panel |  |  |
| Type of standard |  | Choose an item. |
| Unique reference number and title of the proposed new methodology or new methodological tool |  | >> |
| Date when this form was received at UNFCCC secretariat: |  | Click or tap to enter a date. |
| Date of posting in the UNFCCC A6.4 web site for global stakeholder consultation |  | Click or tap to enter a date. |

Information to be completed by the submitter (read before filling the form)

This form is required at the “submission of proposed new methodology or methodological tool” stage and is submitted together with ‘New baseline and monitoring methodology and methodological tool proposal form (A6.4-FORM-METH-001).

Instructions for using this form In using this form, please follow the guidance established in the following documents: - Fill out all relevant sections of the form in clear print or typing; - Provide your input after the >> indicator in the space provided; - Leave blank sections which are found to be not applicable. Formatting Instructions: - Do not modify any part of this form, including headings, logo, format or font; - The form provides the formatted headings which should be used throughout the document; - Please use word equation editor to write equations; - Please format figures, tables and footnotes to update automatically; - Please note the footnotes have a separate format (Times New Roman - size 10). - Please clearly distinguish between proper methodology text, tables and equations and explanatory notes, using the following colour coding: - Methodology text shall be written in black fonts. - Guidance from the UNFCCC is provided in blue fonts and can be deleted. - Explanatory notes shall be written in grey fonts. Please note that explanatory notes are solely for the sake of methodology submission and consideration. Do not include guidance to activity participant in explanatory notes. Please note upon methodology approval, explanatory notes will be deleted.

- Summary and applicability of the baseline and monitoring methodology or methodological tool

Title, submission date and version

Title: Pumped Hydro Storage and Supply of Electricity to the Grid

Submission date: 12th May 2025

Version: 1.0

Submission Date: 13th August 2025

Version: 1.1

Submission Date: 29th August 2025

Version: 1.2

If this methodology or methodological tool is based on a previous submission or an approved Article 6.4 mechanism methodology or methodological tool, please state the reference numbers here. Explain briefly the main differences and their rationale.

This methodology is not derived from any previous submissions or approved methodologies to Article 6.4 mechanism.

Summary description of the methodology or methodological tool, including major baseline and monitoring methodological steps.

This methodology applies to projects that store electrical energy as potential energy at time in the day, convert it back into electricity, and supply it to the grid at later time in the same day. The electricity generation, energy storage, and end-use may occur at the same location or at different locations, as long as all components are connected to the grid.

The methodology is designed to assess emission reductions resulting from time-shifting (or time translation) of electricity from low-emission to high-emission periods, using energy storage systems. Only the emission reductions attributable to the storage element of the system are considered as contributing to the mitigation activity.

When energy storage technologies such as pumped hydro storage (PHS) are combined with renewable energy (RE) sources (e.g., solar or wind), they form a dispatchable clean energy system that displaces fossil fuel-based electricity during peak demand periods or when renewable generation is not available.

The baseline scenario assumes that the renewable electricity is supplied directly to the grid without storage, typically during periods when the grid is already dominated by low-emission sources such as wind, solar, or hydropower. In contrast, the project activity stores this electricity during those low-emission periods and dispatches it later when the grid is primarily powered by high-emission fossil fuel sources—thereby achieving emission reductions through strategic timing.

This methodology covers two types of project configurations:

- In an integrated configuration, the RE generation plant (e.g., solar or wind) is developed in conjunction with the PHS system and directly supplies electricity for water pumping.

- In a non-integrated configuration, electricity used for pumping is sourced from third-party RE generators or through grid-based contractual arrangements.

In both configurations, the methodology requires that electricity used for pumping is primarily renewable and includes safeguards to limit fallback to fossil-based grid electricity.

The methodology employs real-time dispatch analysis to estimate the emission coefficient of the electricity grid in an hour or smaller period ensuring accurate representation of the emission intensity during storage and dispatch periods.

- Proposed new baseline and monitoring methodology or methodological tool

- Introduction

- The following table presents a summary of the key elements of a methodology:

Table 1. Methodology key elements

| Type of GHG mitigation measure(s) | Fuel/feedstock switch Technology switch GHG destruction GHG formation avoidance Engineered carbon dioxide removal Nature based carbon dioxide removal |
| --- | --- |
| Types of mitigation outcomes achieved under this methodology | Emission reductions Removals |
| Are the mitigation outcomes under this methodology at risk of reversal? | Yes No |
| Typical projects eligible under the methodology | Greenfield Grid-connected pumped hydro storage (PHS) systems that convert renewable electricity into stored potential energy of water, which is later released during periods of peak electricity demand, thereby displacing electricity that would otherwise be generated from more GHG-intensive sources. |

- Scope and entry into force

Scope

This methodology provides a standardized approach to quantifying greenhouse gas (GHG) emission reductions from grid-connected energy storage systems that enable time-shifted delivery of renewable electricity.

The methodology supports projects where renewable electricity is stored during periods of low specific grid emissions (tCO₂e/MWh) and later dispatched during high-demand periods, displacing more carbon-intensive electricity generation.

The methodology applies to project activities where:

- Renewable electricity is stored during periods of low grid emission intensity (tCO₂e/MWh); and

- The stored electricity is subsequently dispatched during high-demand periods, displacing more carbon-intensive grid electricity generation.

This methodology applies only to grid-connected energy storage systems that are charged exclusively with renewable electricity, with limited permissible allowance for non-renewable electricity used for pumping.

Entry into force

- [For the UNFCCC secretariat to complete - Leave blank]

Applicability of sectoral scopes

Designated operational entities validating and verifying Article 6.4 activities that use this methodology shall apply sectoral scope(s):

- [For the UNFCCC secretariat to complete – leave blank]

- Definitions

In addition to the definitions contained in the ‘Article 6.4 mechanism Glossary of Terms’, the following definitions apply for the purpose of this methodology:

Grid - A facility that receives electricity generated from the project activity renewable energy power plant with an aim to meet its entire or partial electricity demand.

Installed power generation capacity (or installed capacity or nameplate capacity) - The installed power generation capacity of a power unit is the capacity, expressed in Watts or one of its multiples, for which the power unit has been designed to operate at nominal conditions. The installed power generation capacity of a power plant is the sum of the installed power generation capacities of its power units.

NLDC/SLDC - National or State Load Dispatch Centre’s. The main functions assigned to NLDC/SLDC are:

- Supervision Over the Regional Load Despatch Centres. (RLDC)

- Scheduling and dispatch of electricity over the inter-regional links in accordance with grid standards specified by the authority and grid code specified by Central Commission in coordination with Regional Load Despatch Centres.

- Coordination with Regional Load Despatch Centres for achieving maximum economy and efficiency in the operation of National Grid.

- Monitoring of operations and grid security of the National Grid.

- Coordination for restoration of synchronous operation of national grid with Regional Load Despatch Centres.

- Coordination for trans-national exchange of power.

Open loop - With either an upper or lower reservoir that is continuously connected to a naturally flowing water source such as a river. At least one of the reservoirs connected to a source of natural inflow (Natural Lake, river, river-fed reservoir, the sea)

Power plant/unit - A power plant/unit is a facility that generates electric power. Several power units at one site comprise one power plant, whereas a power unit is characterized by the fact that it can operate independently from other power units at the same site. Where several identical power units (i.e. with the same capacity, age and efficiency) are installed at one site, they may be considered as one single power unit.

Reservoir - A reservoir is a water body created in valleys to store water generally made by the construction of a dam.

Wheeling - Is an electricity transmission service that enables the instantaneous delivery of renewable energy, to a grid without any time lapse between the generation and consumption of electricity, under a power purchase agreement. This includes other similar arrangements such as open access mechanism, if the conditions related to time lapse between generation and consumption of the electricity are satisfied.

- Normative references

This proposed baseline and monitoring methodology is based on the following proposed new methodologies and/or approved or consolidated methodologies:

ACM0002: Grid-connected electricity generation from renewable sources --- Version 22.0

This methodology also refers to the latest approved versions of the following methodological tools:

A6.4-STAN-METH-005 – Addressing leakage in mechanism methodologies (v.01.0)

A6.4-STAN-METH-001 – Application of the requirements of Chapter V.B (Methodologies) for the development and assessment of Article 6.4 mechanism methodologies (v.01.1)

A6.4-STAN-METH-004 - Setting the baseline in mechanism methodologies (v.01.0)

- Applicability

The methodology is applicable under the following conditions:

  - The system stores potential energy in the form of water in a reservoir (stored component) which is subsequently converted back into electricity and supplied to the grid (which may be the same grid or a different one). (Assessed once at validation)

  - The RE generator, storage system owner/operator, and end-user of electricity may be the same entity or separate entities. (Assessed once at validation)

  - The methodology is applicable only where greenhouse gas (GHG) intensity data of the connected electricity grid is publicly available at hourly or finer time intervals for each day of the year, In cases where such data is monitored in real time but released by the grid authority later (e.g., monthly or annually), the methodology remains applicable, if project verification is conducted only after the official release of the data, using the most recent publicly available figures for the relevant monitoring period. (Assessed at each verification)

  - This methodology is applicable to both integrated project activities—where the renewable energy plant and pumped storage system are part of the same project—and non-integrated activities—where renewable electricity is sourced from third-party generators or the grid

  - The RE plant(s) supplying electricity — either as captive assets within an integrated activity or as identified third-party/grid-connected facilities in a non-integrated activity — must be clearly associated with the PHS project and provide dispatchable or intermittent electricity. All major components of the associated RE generation or storage assets must have commenced operation within the past two years. (Assessed once at validation)

  - Any identified RE generators or suppliers, whose operation start date exceeds 180 days prior to the start of the PHS project activity, must demonstrate that they did not have long-term renewable electricity supply contracts with other end users and that their supply has been newly redirected or contractually linked to the PHS Project. (Assessed once at validation)

  - For projects to be eligible under this methodology, one of the following conditions must be satisfied (Assessed once at validation):

- The project is implemented as a greenfield project involving multiple reservoirs, with no alteration in the volume of any reservoir; or

- The project involves the construction of new single or multiple reservoirs, and the power density calculated using Equation (7) is greater than 4 W/m².

  - The project activity shall not be eligible under this methodology for any monitoring period in which the share of fossil fuel-based grid electricity used for pumping exceeds 25% of the total electricity consumed. (Assessed at each verification)

  - Additionally, in any monitoring period where net emission reductions are negative, the project shall not be eligible to claim emission reductions. The negative emission reductions shall be carried forward and deducted from verified emission reductions of subsequent monitoring period(s) until fully compensated (Assessed at each verification)

- Avoidance of double-counting

A paragraph

- (i) Ensure that neither the pumped storage project nor any associated renewable energy plant (whether integrated or third-party) is registered under the A6.4 mechanism or any other crediting mechanism for the same emission reductions

- (ii) Ensure that any renewable electricity used for pumping is not associated with environmental attribute claims (e.g., RECs, I-RECs, GOs) by any other entity; and

- (iii) Include contractual and monitoring safeguards to prevent double issuance, double claiming, or double monetization of emission reductions.

- The above conditions shall be verified by the DOE at the validation stage and during each crediting period renewal.

- Demonstration of alignment with the policies, options and implementation plans with regard to the NDC and LT-LEDS of the host Party and the long-term temperature goal of the Paris Agreement and long-term goals of the Paris Agreement

Activity participants shall provide to the DOE responsible to perform the validation of the Article 6.4 project an assessment, undertaken by the DNA of the host Party, of the activity’s consistency with Decision 3/CMA.3 paragraph 40 (c) and paragraph 27 (a) as part of the host Party’s approval”.

- Activity Boundary

The methodology requires project participants to clearly delineate the project boundary at the time of Project Design Document (PDD) preparation, in accordance with Paragraph 9 of the Appendix to the Baseline Setting Standard (A6.4-STAN-METH-004). The boundary shall include:

- (i) Pumped Hydro Storage (PHS) plant – including all components involved in storing and generating electricity via water reservoirs (e.g., upper and lower reservoirs, turbines, pumps)

- (ii) Associated renewable energy (RE) generation facilities – RE plants supplying electricity for water pumping operations of the PHS plant.

- (iii) All power plants (both renewable and non-renewable) physically connected to the same electricity grid as the PHS project.

- (iv) Electricity grid infrastructure – including transmission and distribution lines through which electricity is either wheeled or directly supplied to/from the PHS project.

- (v) Any auxiliary or backup systems (if applicable) directly involved in the operation of the PHS plant and RE Generation

Table 2. Emissions sources and sinks included in or excluded from the activity boundary

|  | - Source | - GHG |  |  | - Justification / Explanation |
| --- | --- | --- | --- | --- | --- |
| - BASELINE | - CO2 emissions from electricity generation in fossil fuel fired power plants that are displaced due to the project activity | - CO2 | Included - Not included | Controlled Related to - Affected by | - Major Emission Source |
|  |  | - CH4 | Included - Not included | Controlled Related to - Affected by | - Minor Emission Source |
|  |  | - N2O | Included - Not included | Controlled Related to - Affected by | - Minor Emission Source |
|  |  | - ----- | Included - Not included | Controlled Related to - Affected by | - Minor Emission Source |
| - Activity | - Utilization of electricity from grid or from fossil fuel generators by PHS for pumped mode. | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source: electricity may be utilized partially from grid or from fossil fuel generators |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor Emission Source. |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  | - For PHS, emissions of CH4 from the reservoir | - CO2 | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source; included where applicable. |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  | - Auxiliary consumption of electricity in RE plant if electricity is drawn from the grid. | - CO2 | Included Not included | Controlled Related to Affected by | - Main Emission Source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  | - Diesel/Fossil fuel consumption by standby fossil generators in RE and PHS Plant | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  | - CO₂ emissions from construction of PHS facilities | - CO2 | Included Not included | Controlled Related to Affected by | - One-time emissions; excluded for simplicity. Conservative since baseline also omits this. |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Minor Emission Source |
| - LEAKAGE | - Transferred baseline equipment | - CO2 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Resource competition | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Diversion of production | - CO2 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Indirect environmental emissions | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |

- Demonstration of additionality

Regulatory analysis

In accordance with the requirements of the Additionality Standard (A6.4-STAN-METH-003), the project activity must demonstrate that it is not mandated by any applicable legal or regulatory requirement.

The following steps shall be followed by the project proponent to determine regulatory surplus:

- 1.  Host Party approval (through its DNA)

If the host Party approves the project, the approval shall include:

a. Confirmation that, and information on how, the project fosters sustainable development in the host Party.

b. Approval of any potential renewal of the crediting period, if the Party intends to allow the project to continue beyond the first crediting period, where the Party has specified that the crediting periods of A6.4 activities that it intends to host may be renewed pursuant to paragraph 27(b) of the RMPs.

- 2. Identification of Legal and Regulatory Requirements:

The project proponent shall identify and list all applicable legal and regulatory   requirements, including but not limited to:

a. National or sub-national energy policy obligations

b. Environmental regulations (e.g., requirement for Environmental Clearance (EC) or Environmental Impact Assessment (EIA))

c. Land acquisition regulations

d. Water use and reservoir construction permits

e. Grid connection approvals

f.  Construction permits and project development licenses

g. Local government or municipal-level development controls

h. Renewable Purchase Obligations (RPOs), if applicable

- 3. Distinction Between Enabling Approvals and Mandates:

a. Approvals such as environmental clearance, forest clearance, consent to establish (CTE), and consent to operate (CTO) are permitting requirements that allow the project to be developed but do not mandate its implementation.

b. These do not constitute a legal obligation to undertake the project and therefore do not invalidate additionality.

- 4. Demonstration of Regulatory Surplus:

a. It is not required by law to be implemented.

b. It is not undertaken to comply with any binding obligation (e.g., not developed to meet an RPO or grid storage mandate)

c. No regulation or policy compels the project developer to build the facility.

- 5. Evidence and Documentation:

- The project participants shall include in the PDD

a. A table of applicable legal and regulatory requirements and their relevance to the project.

b. A justification of how the project goes beyond these requirements.

c. Where necessary, statements from relevant regulatory bodies or publicly available documentation showing absence of obligation.

Avoidance of locking-in the level of emissions

Lock-in emissions refer to long-term reliance on carbon-intensive infrastructure or energy sources that hinder future decarbonization. PHS projects, when charged using renewable electricity and designed with future-ready flexibility, inherently support a low-emission pathway. To rigorously demonstrate lock-in avoidance, the following structured approach is applied:

Definition of Lock-In Emissions Criteria

Lock-in risks are identified by assessing whether the project:

- Creates dependency on fossil fuels (e.g., uses fossil-based grid electricity for pumping).

- Extends lifespan of high-emission infrastructure (e.g., supports coal-based baseload indirectly).

- Adopts technologies with no decarbonization pathway (e.g., design choices preventing RE integration).

- Triggers market/regulatory shifts favouring fossil fuels (e.g., enables continued fossil dispatch capacity).

Process to Identify Lock-In Emissions Risks

- Step 1: Project Component Analysis

- The following table presents an assessment of potential lock-in risks associated with each major component of the project.

| Component | Lock-In Risk Assessment | Mitigation Strategy & Evidence Required |
| --- | --- | --- |
| Pumping Energy Source | Partial dependency on non-renewable energy for pumping operations. | Limit fossil-based grid input to <25% of annual pumping energy. Submit energy mix projections and grid RE share forecasts. Provide a transition plan toward ≥90% RE use by mid-crediting period. |
| Generation Output | PHS project may indirectly support fossil-based peak demand if not properly timed. | Ensure dispatching of the energy from PHS during peak demand periods to displace fossil-based Peaker plants, promoting RE integration instead. Submit dispatch strategy and historical demand-supply analysis showing fossil displacement effect |
| Technology Selection for Storage and Generation | Use of outdated or less-efficient technologies that limit future performance and emissions reductions. | Implement advanced, efficient technologies. Submit technical specifications and supplier documentation |
| Project Lifetime | Risk of technological obsolescence or degradation in system performance over time. | Design the system for future technological upgrades and improvements. Submit long-term O&M plans, including periodic upgrades and control system modernization |
| Grid Interconnection | Tied to fossil-heavy grids without RE attribution may indirectly support fossil systems. | Submit PPAs (RE Usgae). Provide wheeling agreements or grid mix projections showing increasing RE share. |

- Step 2: Qualitative Checklist for Validation

- The checklist below summarizes the criteria that must be satisfied to confirm that the project avoids lock-in risk, in line with paragraph 29 of A6.4-STAN-METH-004. It also identifies the type of evidence required for validation and verification of these criteria.

| Criterion | Requirement | Evidence Type |
| --- | --- | --- |
| Fossil input for pumping < 25% | Required | Annual energy mix data; metered pumping input records |
| Modular, future-ready technology | Required | Design specs, technology upgradeability declarations |
| No support to fossil Peaker capacity | Required | Dispatch plan, peak demand correlation data |
| Compliance with host country’s decarbonization roadmap | Required | Policy alignment report, grid decarbonization forecast |

Investment analysis, Barrier analysis and Common practice analysis

Investment analysis

The methodology requires that the investment analysis be conducted based on data and assumptions consistent with those used in investment decision-making, reflecting conditions at the start date of the activity.

Type of Investment Analysis: The methodology requires activity participants to apply one of the following types of investment analysis as suitable to the project context, in line with paragraphs 34–36 of A6.4-STAN-METH-003:

- Benchmark analysis: comparing the financial viability of the project to a conservative financial benchmark such as the Weighted Average Cost of Capital (WACC).

- Investment comparison analysis: comparing the project’s financial attractiveness to alternative options.

- Simple cost analysis: for cases where the activity generates no financial return other than from A6.4ERs.

The type of analysis shall be justified by the activity participants, based on the nature of the activity (e.g., whether alternatives are available, whether it is a modification to an existing facility, or a greenfield investment).

Financial Indicators and Carbon Revenue Impact: The financial analysis shall calculate appropriate financial indicators (e.g., IRR, NPV, payback period), both with and without the expected carbon revenues from A6.4ERs. The results must clearly show that:

- The project is not financially viable without A6.4ER revenue (e.g., IRR below benchmark or NPV negative); and

- The inclusion of carbon revenue materially improves the financial viability, such that it becomes attractive to investors.

Data Requirements and Transparency: All financial data (e.g., capital investment, operational and maintenance costs, electricity/fuel revenues, carbon price assumptions, discount rates) must be based on publicly available sources, feasibility studies, or actual investment documents. These shall be transparently documented, with all key assumptions disclosed.

Where relevant, a sensitivity analysis shall be conducted on key variables (e.g., carbon price, input cost, product price) to assess the robustness of the additionality claim.

Barrier analysis

Barrier analysis is not applicable in this case, as the project activity considered under this methodology does not relate to activities outlined in paragraphs 52 and 53 of A6.4-STAN-METH-004—such as those implemented at individual households or by small entities without access to commercial or public third-party finance.

Common practice analysis

This methodology applies the draft Common Practice Assessment Tool (A6.4-MEP07-A01) under the Article 6.4 Mechanism to determine whether the proposed activity is common practice in the applicable geographical area to assess the additionality of the mitigation project activity.

The following parameters shall guide the implementation of the common practice analysis:

| Parameter | Guidance / Value | Justification of the Choice |
| --- | --- | --- |
| Approach used | Approach B – Target market size and market penetration | Approach B is selected due to the limited number of PHS projects in many regions, and because direct comparability (Approach A) may not be feasible. |
| Indicator type | Capacity/output-based - Annual storage capacity (MWh) or installed power (MW) | Output and capacity are the most relevant indicators for comparing energy storage projects and are consistent with regulatory and engineering norms. |
| Data approach | Stock-based – All operational PHS plants started before the PDD publication date or project start date (whichever is earlier) | A stock-based approach captures existing infrastructure and reflects actual deployment at the time of project development. |
| Geographical area | National level – Entire host country | National scope avoids data distortion and reflects the broader energy market context. If a smaller area is used, a justification must be provided. |
| Capacity/output range | ± 50% of the project’s design power output or storage capacity | This ensures that only similar-scale PHS projects are considered, consistent with guidance in the Common Practice Tool. |
| Common practice threshold (Fmax) | 10% – Default value for Approach B | Although the default for Approach B is 3%, a 10% threshold is adopted to reflect the intermediate maturity of PHS technology in some emerging economies. PHS is technically proven but still faces high upfront capital costs, permitting barriers, and long lead times, which limit its wide adoption. The higher threshold aligns with the Tool’s allowance for deviation when justified by technology characteristics and market dynamics, while still preventing over-crediting. |
| Market penetration calculation | Share of operational PHS capacity relative to total installed power capacity in the country or defined region | This captures the penetration of PHS as a distinct energy storage technology and enables differentiation from conventional hydro or battery storage systems. |
| Validation responsibility | DOE to validate all assumptions, data sources, and parameter selections | The DOE is responsible for ensuring transparency and consistency of the common practice analysis at validation and renewal stages. |

The outcome of the common practice analysis, conducted as per the guidance in this methodology and using Tool A6.4-MEP07-A01, shall be used to determine whether the proposed project activity is a widely adopted practice in the applicable geographical area. If the calculated market penetration of the proposed technology, measure, or practice is below the common practice threshold (Fmax), the activity shall be deemed as not common practice and may be considered additional under the Article 6.4 Mechanism.

- Baseline scenario

Selection of the baseline approaches from paragraph 36 of the rules, modalities and procedures

For each emission reduction component, the baseline can be set using one of the following approaches, depending on data availability and project circumstance

Best available technologies that represent an economically feasible and environmentally sound course of action, where appropriate.

An ambitious benchmark approach where the baseline is set at least at the average emission level of the best performing comparable activities providing similar outputs and services in a defined scope in similar social, economic, environmental and technological circumstances.

An approach based on existing actual or historical emissions, adjusted downwards to ensure alignment with paragraph 33 of the RMP.

The baseline scenario refers to present and future portfolio of electricity generation mix connected to the grid. Given the nature of mitigation activity, the baseline is represented by an electricity grid, and these are few for an activity. Hence, benchmark approach is not appropriate. Economically feasible and environmentally sound course of action to temporally flatten the supply or match with the demand. However, such demand side management measures and its impact on baseline emissions is appropriately captured by the chosen approach.   The baseline approach is based on actual or historical emissions, as this reflects the electricity mix that would have operated in the absence of the project. The grid would continue to dispatch electricity from existing or planned sources based on merit order and system demand. This approach is appropriate given the availability of historical dispatch data and the lack of mandatory storage policies or similar projects.

Application of the selected approach, prior to implementation of a downward adjustment

Procedure for the identification of the baseline scenario

The baseline scenario is the continued supply of electricity to the grid using existing grid-connected sources and expected capacity additions following historical trends and merit order dispatch. In the absence of the proposed pumped storage project, demand would be met by the marginal generating units - typically fossil fuel-based plants or combinations of coal, gas, and renewable energy - depending on the grid load and dispatch pattern.

This scenario is to be justified through:

- Actual operational conditions and dispatch behaviour of the grid.

- National regulations or mandates to use of pumped storage or similar technologies to meet peak load

- Alignment with recent historical grid performance and does not assume speculative shifts in generation mix.

- Consistent with the baseline approach selected (Option 3) and relies on actual dispatch-based emissions, adjusted downward where appropriate.

Calculation of baseline emissions prior to downward adjustment

To accurately reflect the marginal emissions displaced by the project activity, it is essential to analyse the hourly variation in the electricity generation mix of the connected grid. This approach ensures that the emissions factor used corresponds to the actual fossil-intensive or renewable composition of the grid at the specific time when the project dispatches electricity or avoids grid withdrawals. The steps below outline the calculating the hourly baseline emissions over the project monitoring period:

Step 1: Collect Real-Time Data for the Project Period

- Gather real-time or periodic data for electricity generated and dispatched during each hour of the operational period. This should include:

- Hourly electricity generation from the project activity (MWh) or any contributing sources.

- Hourly emissions data (in tons of CO2e) for each generating source in the grid.

Step 2: Identify the Contribution of Different Generation Sources

- Identify the generation sources that contribute to meeting the grid demand during each hour. This can include renewable sources (e.g., Hydro, wind, solar), fossil fuel-based plants (e.g., coal, gas), and others.

- For each hour, determine the contribution of each source in terms of electricity generation (MWh) and emissions (tons of CO2e).

Step 3: Calculate Hourly Emission Factor (EF) for Each Hour

- For each hour h during which the project activity supplies electricity to the grid, and for each hour during which electricity is drawn from the grid (excluding renewable energy sources), calculate the corresponding hourly emission factor.

Step 4: Monitor Hourly Emissions and Adjust as Needed

- Continue to calculate the emission factor for each hour in real-time or at regular intervals, based on the generation mix for each hour.

- Ensure that the emissions are being tracked accurately, and adjustments may be required if the energy mix changes significantly over time.

Step 5: Calculate the Emission Factor at an hour “h” on a day “d” in year “y”

- Calculate the Emission Factor at a specific hour of the day for the entire monitoring period. This will reflect the marginal emissions associated with electricity generation during that specific time, accounting for fluctuations in the energy mix.

In the baseline scenario, the electricity generated by the project activity related to renewable sources is directly supplied to the electricity grid (instead of storing the same). Accordingly, the net baseline emissions at any hour of the day and year are as follows:

Equation (1)

BERy = d=1365 h=124(EGh,d,yDelivered*yh,d,y * EFDD,Delivered,h,d,y )*AFy

- With:

Equation (2)

AFy=1-  y- YcurrentYNet Zero - Ycurrent

- Where:

| BER,y | = | Baseline Scenario emission reductions at an hour ‘h’ on a day ‘d’ in year ‘y’ (t CO2/hr) |
| --- | --- | --- |
| EGh,d,yDelivered | = | Electricity delivered to grid at hour h, day d in year y (MWh) |
| EFDD,Delivered,h,d,y | = | Grid emission factor for delivered electricity at hour h, day d, in year y (tCO₂/MWh) applicable during the electricity delivery period |
| yh,d,y | = | Fraction of delivered electricity by the project activity that is renewable at hour h, day d, in year y (0 ≤ y ≤ 1) |
| AFy | = | Adjustment Factor |
| Ycurrent | = | Project start / reference year |
| YNetZero | = | Host-country net-zero target year |
| y | = | Year of issuance for which the emission reductions are claimed |

Assumptions, Parameters, and Data Requirements (as per Paragraph 11 of the Baseline Standard):

- This section provides the assumptions, parameters, data sources, and data quality requirements used for determining the baseline scenario and calculating baseline emissions, in accordance with Paragraph 11 of the Baseline Standard.

- 1. Key Assumptions

- The grid dispatch follows merit-order based on system demand and generator availability.

- No regulatory mandate or policy requires storage technologies (e.g., pumped storage) during the baseline period.

- Grid emission intensities vary hourly and are treated as time-series data, not single-point values.

- 2. Data Sources

- Grid Emission Factors: Central Electricity Authority (CEA) published hourly grid emission factors, or real-time data from SLDCs and POSOCO.

- Project Electricity Flows: Collected via calibrated meters with secure digital logging.

- Generation Mix Data: Authorized SCADA/grid operator data providing hourly generation by source type.

- National Policy References: Used to confirm absence of regulations mandating pumped hydro during the baseline period (e.g., MoP notifications, NDCs, LT-LEDS).

- 3. Data Quality and Credibility Requirements

- Vintage: Hourly grid emission data shall be based on real-time or periodically updated dispatch data from credible sources (e.g., SLDCs, POSOCO, CEA). Only time-aligned data for the relevant monitoring period shall be used to ensure accuracy of marginal emission calculations.

- Availability: All parameters must be measurable or reliably estimated based on documented procedures.

- Credibility: Prefer public, government, or utility-published data. Where not available, justification must be provided along with estimation procedures.

Identification of Uncertainties: In accordance with Paragraphs 13, 14, and 15 of Appendix 1 to the Baseline Standard, this methodology systematically assesses the uncertainty associated with the quantification of baseline emissions. A conservative approach is applied based on the level of identified uncertainty to ensure the environmental integrity of the credited emission reductions.

Identification of Sources and Level of Uncertainty: The key sources of uncertainty in the quantification of emission reductions are identified as follows:

- 1.  Uncertainty in Grid Emission Factors (UEF): This is the primary source of uncertainty. The emission factors are derived from historical grid dispatch data, which may have inaccuracies due to measurement errors, reporting delays, or incomplete data for all generation units. The uncertainty associated with national grid emission factors is typically moderate to high.

- Level of Uncertainty: It needs to be assessed considering accuracy of historical grid dispatch data - measurement errors, reporting delays, or incomplete data and the complexity of aggregating data from diverse generation sources. In case such estimate is not feasible, the uncertainty in Grid Emission factor can be considered 10%

- 2.  Uncertainty in Activity Data (UActivity): The net electricity generation of the PHS plant are measured by meters. While modern meters are accurate, there is still a small degree of measurement uncertainty.

- Level of Uncertainty: Assessed at 1%. This is based on the standard accuracy of Class 0.5 electricity meters (±0.5%) and potential data transmission or logging errors. In case, project proponent is using more accurate electricity meters, the same can be calculated and used

- 3.  Uncertainty in the Baseline Scenario Assumption: The baseline relies on the assumption that the historical dispatch pattern (merit order) will continue. Unforeseen events, such as the unplanned shutdown of a large fossil fuel plant or a significant policy shift, could alter this pattern, introducing uncertainty into the representativeness of the historical data.

- Level of Uncertainty: Assessed as “qualitatively high” but difficult to quantify precisely. The methodology mitigates this using multi-year historical data and the requirement for the most conservative baseline option.

Calculation of Combined Uncertainty:

- The combined uncertainty (Ucombined) of the annual emission reductions (ERy) is calculated using the root sum of squares method for the two primary quantifiable uncertainties:

- Ucombined = √(U2EF + U2Activity                                  Equation (3)

- Where:

| UEF | = | Uncertainty in Grid Emission Factor = 10% (Default) |
| --- | --- | --- |
| UActivity | = | Uncertainty in Net Generation Measurement = 1% (Default) |
| Ucombined | = | Combined uncertainty of the annual emission reductions |

Ucombined= √(0.10² + 0.01²) = √(0.01 + 0.0001)

Ucombined = √0.0101 ≈ 0.10 or 10%

Application of Conservativeness

- To ensure the environmental integrity of the baseline emission reductions, a Conservativeness Factor (CF) is applied to the calculated annual baseline emission value. The CF is determined based on the level of combined uncertainty.

Given the high combined uncertainty of approximately 10%, a Conservativeness Factor of CF = 90% (in case both default factors for estimation of uncertainty is used) is applied.

The conservative baseline emission is calculated as:

- BERcons,y= BERy*CF                                       Equation (4)

- Where:

| BERcons,y | = | Conservative baseline emission |
| --- | --- | --- |
| CF | = | Conservativeness Factor |

This approach ensures that the reported baseline emission is conservative and robust, directly addressing the systematic uncertainty identified in the baseline emission quantification as required by the Baseline Standard.

Calculation of the downward adjusted baseline

In this methodology, the BAU and Baseline is not a number but a set of 7860 or more numbers i.e. emission intensity of the relevant grid at each hour (or a smaller duration) in a day during the 365 days.  Accordingly, comparison of BAU and Baseline is not possible to determine downward adjustment. A conservative estimate of emission reductions can be arrived at by following the procedure below:

  - Compute the annual emission reductions attributable to the mitigation activity using BAU and Baseline emissions.

  - Chose lower amongst the two numbers representing a conservative value.

- Therefore:

BEy = min (BAUy,BERCons,y)                                  Equation (5)

Identification of the conservative BAU scenario

This section provides a methodological approach to identify the Business-as-Usual (BAU) scenario for the proposed project activity in accordance with the “Baseline Setting Standard for Article 6.4 Mechanism Activities” (A6.4-STAN-METH-004). The selected BAU scenario represents what would have occurred in the absence of the Article 6.4 activity and serves as the basis for baseline emissions estimation and subsequent downward adjustment, where applicable.

Identification of the Pre-Project Scenario: In accordance with paragraph 10 of A6.4-STAN-METH-004, activity participants shall describe the pre-project situation that existed prior to the implementation of the project activity. This includes:

  - The source(s) of electricity generation and its temporal pattern, that is meeting the demand during different time of the day (e.g., fossil fuel plants, RE, grid imports).

  - Any storage capacity or backup systems operational prior to the implementation of the project at the generation facility or the electricity grid system.

This description shall be based on verifiable data and reflect the conditions in the host country or region within the three most recent years prior to the project’s implementation or start date.

In line with Section 6.1 of the “Baseline setting in mechanism methodologies” standard (A6.4-STAN-METH-004), the methodology identifies the most plausible and conservative Business-as-Usual (BAU) scenario to represent what would have occurred in the absence of the Article 6.4 activity

The following potential BAU scenarios are recognized under the standard:

- Continuation of the historical situation (pre-activity scenario).

- Establishment of an economically viable technology and/or practice that would have been implemented in the absence of the project.

- A combination of options (a) and (b), such as continuation of existing practice followed by equipment replacement over time.

- Any other justified scenario that is consistent with the standard’s requirements and is conservative.

This methodology adopts Scenario (a) - Continuation of the historical situation (pre-activity scenario) - as the most plausible and conservative BAU scenario, considering the nature of grid-connected power systems in many countries, where fossil-based and mixed-generation sources remain prevalent and are economically viable.

Activity participants are required to justify the applicability of Scenario (a) during project validation using evidence such as:

- (a) Historical electricity generation and dispatch data,

- (a) Grid reliability trends

- Note: If a standardized baseline is available and applicable to the host country or grid region, the BAU scenario may be aligned with it in accordance with A6.4-STAN-METH-004.

Procedure for Business-as-Usual Estimation

- The Business as Usual (BAU) scenario represents the emissions that would have occurred in the absence of the pumped hydro storage (PHS) project. Establishing this scenario is essential to demonstrate the additionality of the project, ensuring that the emission reductions achieved are not part of a baseline or expected development trajectory.

To estimate the BAU emissions, the following procedure is adopted:

  - Define the Project Output:

- The PHS project provides electricity during peak demand periods by storing energy and displacing more carbon-intensive generation. In the absence of the project, this electricity would be supplied by the existing grid mix, primarily fossil fuel-based sources.

  - Historical Dispatch Data Analysis:

- A multi-year analysis (typically 3–5 years) of historical hourly dispatch data is conducted to assess how the grid has met demand in similar timeframes. This helps identify the marginal units (i.e., the units that respond to changes in demand) and their emission characteristics.

  - Calculate Hourly Emission Trends:

- The hourly emission factors are computed for each hour of each day across the selected historical period. This provides insight into the marginal emissions at different times, accounting for fluctuations in fuel mix, load demand, and renewable generation.

  - Establish Projected Margin Emission Factors:

- This historical hourly emission factors’ trend is extrapolated to arrive at projected margin hourly emission factors. This projection reflects expected marginal emissions that would occur without the project, assuming current grid dynamics and trends continue.

  - Apply Projected Margins to Project Output:

- The estimated project electricity supply (on an hourly basis) is then multiplied by the corresponding projected margin emission factor to estimate the total BAU emissions. This ensures that the BAU scenario reflects hourly emissions likely to occur, if the present growth trends of RE generation assets, demand patterns, grid management rules and regulations continue.

Calculation of the conservative BAU emissions

Equation (6)

For a PACM project, the Business-as-Usual (BAU) scenario typically reflects the emissions resulting from the conventional grid mix used to meet electricity demand. Accordingly, BAU emissions are estimated by assessing the emission intensity of the existing and projected energy sources supplying the grid.

- BAUy = d=1365 h=124(EGh,d,yDelivered*yh,d,y * EFPM,h,d,y )

- Where:

| BAUy |  | = | BAU emissions for year y, based on historical trends and projections of grid-connected power plants t CO2 |
| --- | --- | --- | --- |
| EGh,d,yDelivered |  | = | Electricity delivered to grid at hour h, day d in year y (MWh) |
| EFPM,h,d,y |  | = | Projected Margin grid emission factor at hour h, day d, in year y (tCO₂/MWh) |
| yh,d,y |  | = | Fraction of delivered electricity by the project activity that is renewable at hour h, day d, in year y (0 ≤ y ≤ 1) |

Steps to Calculate Projected Margin Emission Factor (EFPM,h,d,y):

- The Projected Margin Emission Factor (EFPM) represents the anticipated marginal emission intensity of the electricity grid for a given hour and day, based on historical dispatch patterns. It is calculated by analysing historical emission factors on an hourly basis over a period of 3 to 5 years. This approach captures both intra-day (hourly) and seasonal (daily/monthly) variations in grid emissions.

This methodology assumes that recent historical trends in marginal emission intensity are indicative of future performance, allowing for a forward-looking estimate of grid emissions that better aligns with actual operational conditions.

- EFPM,h,d,y = EFh,d,N +​ (1N-1)y=2N(EFh,d,y- EFh,d,y-1)             Equation (7)

- Where:

| EFh,d,y | = | Historical emission factor for hour h, day d, and year y (t CO2/MWh) |
| --- | --- | --- |
| EFh,d,N | = | Emission factor for the latest historical year for hour h, day d, year y (in tCO₂/MWh) |
| N | = | Number of historic years considered for the calculation (typically 3 to 5 years) |

Comparison of the downward adjusted baseline and the conservative business-as-usual baseline

As per the section 9 of the baseline standard, this section outlines the methodology for calculating the difference between conservative business-as-usual emissions and downward adjusted baseline emissions for energy storage systems

Each year within the crediting period, the difference between conservative business-as-usual baseline emissions and downward adjusted baseline emissions is calculated as:

- ∆Ey= BAUy- BERCons,y                                 Equation (8)

- Where:

| ∆Ey | = | Difference in emissions for year y (tCO₂e/yr) |
| --- | --- | --- |

- Activity scenario

Calculation of activity emissions

Energy storage systems may generate activity emissions due to energy losses during charge and discharge cycles, as well as the use of non-renewable electricity for charging. In the case of pumped hydro storage (PHS) projects, additional emissions may result from water evaporation from the reservoir.

Following project emissions are taken into consideration PEFF,y , PEHP,y, PEgrid,y, PEmw,y

- Where:

| PEFF,y | = | Project emissions from fossil fuel consumption in year y (t CO2e/yr). |
| --- | --- | --- |
| PEHP,y | = | Project emissions from water reservoirs of pumped hydro storage (PHS) plants in year y (t CO2e/yr). |
| PEgrid,y | = | Project emissions from electricity drawn from the grid to charge the storage system in year ‘y’ (tCO₂e/year). |
| PEmw,y | = | Project emissions associated with pumping makeup water to the reservoir to compensate for evaporation losses. |

Steps to estimate for activity emissions, are given below:

- Step 1. Emissions from fossil fuel consumption at project site (PEFF,y)

PEFF,y= f(Qf,y *EFf,y*NCVf)

Equation (9)

- Where:

| PEFF,y | = | Project emissions from fossil fuel consumption in year y (t CO2e/yr) |
| --- | --- | --- |
| Qf,y | = | Quantity of fuel ‘f’ consumed during year ‘y’ (Gg/yr) |
| NCVf | = | Net Calorific Value of fuel ‘f’ (TJ/Gg) |
| EFf,y | = | Emission factor for fuel ‘f’(t CO2e/TJ) |

- Step 2. Emissions from water reservoirs of pumped hydro storage (PHS) projects (PEHP,y)

- The power density (PD) of the project activity is calculated as follows

| PD=CapPJ-CapBLAPJ-ABL | Equation (10) |
| --- | --- |

Where:

| PD | = | Power density of the project activity (W/m2) |
| --- | --- | --- |
| CapPJ | = | Installed capacity of the pumped hydro storage (PHS) project after the implementation of the project activity (W) |
| CapBL | = | Installed capacity of the pumped hydro storage (PHS) project before the implementation of the project activity (W). For new PHS, this value is zero |
| APJ | = | Area of the single or multiple reservoirs measured in the surface of the water, after the implementation of the project activity, when the reservoir is full (m2) |
| ABL | = | Area of the single or multiple reservoirs measured in the surface of the water, before the implementation of the project activity, when the reservoir is full (m2). For new reservoirs, this value is zero |

- For pumped hydro storage (PHS) project activities that result in new single or multiple reservoirs and activities that result in the increase of single or multiple existing reservoirs, project participants shall account for CH4 and CO2 emissions from the reservoirs, estimated as follows:

- For PHS, PD of the entire project is calculated as follows:

| PD=Cap PJ,iAPJ,j | Equation (11) |
| --- | --- |

Where:

| i | = | Individual power plants included in PHS |
| --- | --- | --- |
| j | = | Individual reservoirs included in PHS |

- If the power density of the project activity using equation (11) is greater than 4 W/m2 and less than or equal to 10 W/m2:

| PEHP,y,h=EFRes×EGh,d,yDelivered1000 | Equation (12) |
| --- | --- |

Where:

| PEHP,y | = | Project emissions from water reservoirs (t CO2e/hr) |
| --- | --- | --- |
| EFRes | = | Default emission factor for emissions from reservoirs of hydro power plants (kg CO2e/MWh) |

- If the power density of the project activity is greater than 10 W/m2:

| PEHP,y=0 | Equation (13) |
| --- | --- |

- Step 3. Emissions from charging the storage using the power from the grid (PEgrid,y)

- Under normal conditions, the charging of storage system is performed with the renewable electricity using the transmission infrastructure of the grid system. Pumped Hydro Storage (PHS) plants may utilize grid electricity in addition to the electricity supplied by the renewable power plant (EGgrid,h,d,y). It uses the electricity drawn from the grid in delivering the grid balancing services to accommodate the addition of renewables to the grid. Pumped Hydro Storage (PHS) projects primarily function as enabling technologies that facilitate the integration of variable renewable energy sources, thereby enhancing grid stability and reducing overall system reliance on fossil fuel-based generation aligning with the broader intent of Article 6 to promote innovative, system-enhancing solutions that contribute to global decarbonization objectives. Accordingly, the activity emissions arising from the use of grid electricity by the project activity be computed.

Equation (14)

In cases where PHS utilizes grid electricity, the corresponding project emissions (PEgrid,y) shall be calculated. Furthermore, the project participant(s) should compensate in full any negative emissions reductions which may arise from power consumption from the grid

PEgrid,y = d=1365 h=124EGh,d,yReceived*xh,d,y *EFDD,h,d,Recived,y

Where:

| PEgrid,y | = | Project emissions due to electricity used for charging the storage system from the electricity grid (Fossil based) in year ‘y’(t CO2e/yr) |
| --- | --- | --- |
| EGh,d,yReceived | = | Electricity used for pumping water to the upper reservoir pumping at hour h, day d, in year y (MWh) |
| EFDD,h,d,Recived,y | = | Grid emission factor for received electricity for pumping water to the upper reservoir at hour h, on day d, in year y (tCO₂/MWh), applicable during the electricity receiving period. |
| xh,d,y | = | Fraction of received electricity used for pumping water to the upper reservoir for pumping that is non-renewable (0 ≤ x ≤ 1) |

- Step 4. Emissions due to makeup water pumped to the reservoir due to evaporation  (PEmw,y)

- Consider the project emissions due to make up water only if non-RE power is being used in pumping to the reservoir. And is calculated as follows

- PEmw,y = Vevap,y* SECpump*EFDD,h,d,Recived,y               Equation (15)

Where:

| PEmw,y | = | Project emissions due to makeup water pumped to the reservoir due to evaporation(tCO2/year) |
| --- | --- | --- |
| Vevap,y | = | Volume of water lost through evaporation over the year (m3/year) |
| SECpump | = | Specific Energy Consumption for Pumping (MWh/m3) |

Activity emissions are calculated as follows:

| PEy=PEFF,y+PEHP,y+PEgrid,y+ PEmw,y | Equation (16) |
| --- | --- |

- Where:

| PEy | = | Project emissions in year y (t CO2e/yr) |
| --- | --- | --- |
| PEFF,y | = | Project emissions from fossil fuel consumption in year y (t CO2e/yr). |
| PEHP,y | = | Project emissions from water reservoirs of pumped hydro storage (PHS) plants in year y (t CO2e/yr). |
| PEgrid,y | = | Emissions from charging the storage using the power from the grid or from fossil fuel electricity generators |
| PEmw,y | = | Project emissions due to makeup water pumped to the reservoir due to evaporation. |

- Leakage

Identification of leakage emission sources

In accordance with the requirements of the Standard for Addressing leakage in mechanism methodologies (A6.4-STAN-METH-005) (Version 1.0), this methodology identifies all potential sources of leakage relevant to eligible project activities under this methodology.

Leakage emissions sources in includes:

- Transfer of Baseline Equipment

- Competition for Resource Use

- Diversion of Existing Production or Outputs

- Environmental GHG Release

Avoidance or minimization of leakage

Transfer of Baseline Equipment: If project implementation involves the relocation or repurposing of low-emission infrastructure (e.g., hydro turbines) from existing applications where they were displacing fossil-based energy, the resulting emissions increase at the original location is considered leakage.

- Applicability: This methodology is limited to greenfield projects. There is no transfer or repurposing of baseline equipment from other low-emission applications.

- Leakage Consideration: Leakage emissions associated with equipment transfer are considered zero.

Competition for Resource Use: Project activities that divert or reduce the availability of shared resources—such as renewable energy, water for pumping reservoirs, or land for reservoir construction—may lead to indirect environmental impacts or emissions, which are treated as leakage.

- Applicability: If the project proponent demonstrates that the water used for pumping is available in surplus in the project region, leakage from water diversion is considered negligible.

- Leakage Consideration: If water or other shared resources are diverted from other uses, the associated leakage emissions shall be calculated according to the methodology.

Diversion of Existing Production or Outputs: The methodology applies only to greenfield projects. There are no changes in the type(s) of output or level(s) of service provided compared to the baseline scenario.

- Applicability: Methodology applies only to greenfield projects. Since there is no existing production or service level, no diversion of production or outputs is possible.

- Leakage Consideration: Diversion of existing production or outputs is not a potential source of leakage.

Environmental GHG Releases (Land Use Change and Displacement): Environmental emissions resulting from construction, operation, or disposal of storage infrastructure that fall outside the direct project boundary but are causally linked to the project are considered leakage.

- Applicability: Applies to greenfield projects involving land use change for reservoir or infrastructure construction. All indirect emissions outside the project boundary caused by such changes are considered leakage.

- Leakage Consideration: Includes:

- Methane emissions from flooded biomass or submerged vegetation in new PSH reservoirs.

- CO₂ emissions from land-use change, such as deforestation for reservoir or infrastructure development.

- These indirect environmental emissions are calculated and included as leakage to ensure that all causally linked emissions outside the project boundary are properly accounted for.

Addressing leakage emissions

Where leakage is identified as significant, it must be quantified using conservative assumptions and, where available, default emission factors or project-specific data.

- The total leakage emissions (LEy) for a given year shall be calculated as:

- LEy​=LEequip,y​+LEres,y​+LEdiv,y​+LEenv,y                                  Equation (17)

- Where:

| LEy |  | = | Total leakage emission in a year y (tCO2) |
| --- | --- | --- | --- |
| LEequip,y |  | = | Leakage from transferred baseline equipment in a year y (tCO2) |
| LEres,y |  | = | Leakage from resource competition in a year (tCO2) |
| LEdiv,y |  | = | Leakage from diversion of production in a year (tCO2) |
| LEenv,y | = |  | Leakage from indirect environmental emissions in a year (tCO2) |

Step 1: Transfer of Baseline Equipment (Applicable to brownfield project activities):

- This methodology applies only to greenfield projects; therefore, no transfer of baseline equipment occurs, and leakage emissions from such transfer are considered zero.

Step 2: Competition for Resource Use:

- There is potential competition for resource use, as the project is a greenfield activity utilizing resources (electricity and water) that could otherwise be employed in alternative activities; therefore, leakage emissions from competing resource use are considered. However, since the renewable energy generation is greenfield, less than two years old, and has not entered any long-term supply contracts during its brief operational period, such leakage emissions are not applicable.

- For diverted water resources, the associated leakage emissions shall be estimated as, If the project proponent demonstrates that the water used for pumping is available in surplus quantity in the project region, leakage associated with the diversion of water usage shall be considered negligible.

- However, if the water is diverted from other usage, leakage emissions associated with such diversion shall be calculated as follows

LEWater,y=Vdiv,y*ECalt,y* EFOM,y                                    Equation (18)

- Where:

| LEWater,y | = | Leakage emissions from diverted water usage (tCO₂e/year) |
| --- | --- | --- |
| Vdiv,y | = | Volume of water diverted from other uses (m³/year) |
| ECalt,y | = | Energy consumption per unit of water for the alternative supply (MWh/m³) |
| EFOM,y | = | Operating Margin Grid Emission Factor (tCO₂e/MWh) |

Step 3: Diversion of Existing Production or Outputs

- As the project activity is a greenfield installation, the type(s) of output and level(s) of services provided do not change compared to the baseline scenario. Therefore, leakage from diversion of existing production or outputs - where such changes could lead to increased fossil-based generation elsewhere - is not applicable.

Step 4: Environmental GHG Release

- Leakage emissions are indirect greenhouse gas (GHG) emissions that occur outside the project boundary because of the project activity.

- For projects where the new reservoir floods only areas previously classified as water bodies (e.g., rivers), methane emissions from submerged vegetation and land-use change emissions shall be excluded from leakage calculations, as no significant additional emissions occur.

- However, if the reservoir floods areas not previously classified as water bodies (e.g., forests, croplands, grasslands, or settlements), the following potential sources of leakage emissions shall be considered and quantified as:

LEenv,y= MEveg,y + LUCEmissions,y	                           Equation (19)

- Methane released from submerged vegetation (MEveg)

For greenfield projects where large reservoirs are built, methane release can be a significant source of indirect emissions. The emissions from submerged vegetation are typically related to the area of land flooded by the reservoir.

- MEveg,y = Aflooded​ * EFmethane​ * GWPCH4 			 Equation (20)

- Where:

| MEveg,y | = | Methane Emissions from Submerged Vegetation in a year y |
| --- | --- | --- |
| Aflooded​ | = | Area of land flooded by the reservoir (in hectares). |
| EFmethane | = | Methane emission factor for the flooded land (tonCH₄/hectare/year). |
| GWPCH4 | = | Global Warming Potential of Methane |

- Land Use Change (LUC) Emissions (Direct and Displacement-Related) (LUCEmissions)

- LUCEmissions,y = LUCdirect,y + LUCdisp,y                                            Equation (21)

- Where:

| LUCEmissions,y | = | Total land use change emissions in year y (tCO₂e) |
| --- | --- | --- |
| LUCdirect | = | Emissions from direct land-use change in year y (tCO₂e) |
| LUCdisp | = | Leakage from displaced activities in year y (tCO₂e) |

a) Direct Land Use Change Emissions

These are emissions arising from the conversion of existing land (e.g., forest, grassland) to reservoirs or project infrastructure.

LUCdirect,y = ∑Ai * EFi * (1+Ri)                                                 Equation (22)

- Where:

| Ai | = | Area of land use change type i (ha) |
| --- | --- | --- |
| EFi | = | Emission factor for land use type (e.g., tCO₂/ha/year) |
| Ri | = | Risk or uncertainty adjustment factor for type i |

b) Leakage from Displacement of Pre-Project Activities

If any economic activities (e.g., agriculture, grazing, settlements) occurred on the land prior to flooding, and these activities are relocated to other areas (e.g., forests), the resulting emissions from such secondary land-use change must be included.

LUCdisp,y= ∑Adis,j * EFdis,j * (1+Rdis,j)                             Equation (23)

- Where:

| Ai | = | Area required to continue displaced activity j (ha) |
| --- | --- | --- |
| EFi | = | Emission factor for land use conversion caused by displacement (tCO₂e/ha/year) |
| Ri | = | Risk/uncertainty adjustment factor for activity j |

- Emission reductions

For each year of the crediting period, emission reductions are calculated by applying the baseline, activity, and leakage formulations already defined, using monitored data and verified emission factors

Emission reductions are calculated as follows:

| ERy= BEy- PEy - LEy | Equation (24) |
| --- | --- |

- Where:

| ERy | = | Net Emission Reduction’s due to storage in year ‘y’ (t CO2e/yr) |
| --- | --- | --- |
| PEy | = | Project Emissions in year y (t CO2e/yr) |
| LEy | = | Leakage Emissions in year y (t CO2e/yr) |

- Data and parameters not monitored

| Data/parameter | EFh,d,N |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for the latest historical year for hour h, day d, year y |  |  |  |
| Data unit | tCO2/MWh |  |  |  |
| Equations referred | Equation 7 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The emission factor is estimated from historical hourly grid dispatch and emissions data. The calculation involves marginal emission factors over a baseline period (typically 3–5 years). |  |  |  |
| Treatment of uncertainty | The historical emission factor is based on measured grid dispatch and emission data. Uncertainty arises from temporal variability in generation, reporting errors, and assumptions regarding representativeness of historical years. To address this, a multi-year average (typically 3–5 years) is used, smoothing out anomalous fluctuations. Regular updates and cross-checking with national or regional datasets reduce bias and ensure robustness of the emission factor. |  |  |  |
| Additional comments | The projected margin is used to represent expected marginal emissions from fossil-based generation displaced by the project. This value must be updated periodically (e.g., every crediting period) using the most recent 3–5 years of data. |  |  |  |

| Data/parameter | N |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Number of historic years considered for the calculation of BAU |  |  |  |
| Data unit | No’s |  |  |  |
| Equations referred | Equation 7 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The emission factor is estimated from historical hourly grid dispatch and emissions data. The calculation involves marginal emission factors over a baseline period (typically 3–5 years). |  |  |  |
| Treatment of uncertainty | Uncertainty in the number of historical years considered is addressed by selecting a representative range (3–5 years) that captures variability in grid emissions |  |  |  |
| Additional comments | This value must be updated periodically (e.g., every crediting period) using the most recent 3–5 years of data. |  |  |  |

| Data/parameter | YNet Zero |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Host-country net-zero target year. |  |  |  |
| Data unit | Year |  |  |  |
| Equations referred | Equation 2 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Sourced from the host country's Nationally Determined Contributions (NDCs), official climate policy declarations, or long-term climate strategies submitted to the UNFCCC, which explicitly state the target year for achieving net-zero GHG emissions. |  |  |  |
| Treatment of uncertainty | Uncertainty is associated with policy implementation and potential revisions to NDC targets. To mitigate this, only official, publicly declared targets from credible government or UNFCCC sources are used |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Ycurrent |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Project start / reference year |  |  |  |
| Data unit | Year |  |  |  |
| Equations referred | Equation 2 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determined from project documentation and records indicating the commissioning date or the start of the crediting period. Verified using official project registration documents, commissioning certificates, or other authoritative records. |  |  |  |
| Treatment of uncertainty | Project start year is verified against official documentation. Uncertainty is minimal; any ambiguity is resolved through cross-validation with commissioning certificates or registration documents. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Year of issuance for which the emission reductions are claimed |  |  |  |
| Data unit | Year |  |  |  |
| Equations referred | Equation 2 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determined from project documentation and records indicating the commissioning date or the start of the crediting period. Verified using official project registration documents, commissioning certificates, or other authoritative records. |  |  |  |
| Treatment of uncertainty | The issuance year is based on project records and official documentation. Errors are minimized by using verified project schedules. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFf,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | CO₂ emission factor of the fossil fuel type used, indicating how much CO₂ is emitted per unit of energy content. |  |  |  |
| Data unit | tCO₂/TJ |  |  |  |
| Equations referred | Equation 9 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default emission factors will be sourced from the 2019 refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, unless national-specific factors are available and more accurate. |  |  |  |
| Treatment of uncertainty | Default emission factors from IPCC guidelines have inherent variability depending on fuel composition |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | NCVf |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Net calorific value of the fuel – energy content per unit of fuel |  |  |  |
| Data unit | TJ/tonne |  |  |  |
| Equations referred | Equation 9 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default emission factors will be sourced from the 2019 refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, unless national-specific factors are available and more accurate. |  |  |  |
| Treatment of uncertainty | Using default IPCC values or country-specific factors mitigates bias, and uncertainty is reduced. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | CapPJ |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Installed capacity of the pumped hydro storage (PHS) project after the implementation of the project activity |  |  |  |
| Data unit | Watt |  |  |  |
| Equations referred | Equation 10 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determine the installed capacity based on manufacturer’s specifications or recognized standards |  |  |  |
| Treatment of uncertainty | Capacity uncertainty arises from manufacturer tolerances or operational constraints. Manufacturer specifications and verified project documentation are used. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | CapBL |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Installed capacity of the pumped hrdro storage project before the implementation of the project activity |  |  |  |
| Data unit | Watt |  |  |  |
| Equations referred | Equation 10 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determine the installed capacity based on manufacturer’s specifications or recognized standards |  |  |  |
| Treatment of uncertainty | Capacity uncertainty arises from manufacturer tolerances or operational constraints. Manufacturer specifications and verified project documentation are used. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | APJ |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Area of the single or multiple reservoirs measured in the surface of the water, after the implementation of the project activity, when the reservoir is full |  |  |  |
| Data unit | m2 |  |  |  |
| Equations referred | Equation 10 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Measured from topographical surveys, maps, satellite pictures, etc. |  |  |  |
| Treatment of uncertainty | Measurements from surveys or satellite imagery are cross-checked, and conservative estimates are applied to account for temporal variation. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | ABL |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Area of the single or multiple reservoirs measured in the surface of the water, before the implementation of the project activity, when the reservoir is full |  |  |  |
| Data unit | m2 |  |  |  |
| Equations referred | Equation 10 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Measured from topographical surveys, maps, satellite pictures, etc |  |  |  |
| Treatment of uncertainty | Baseline reservoir area is similarly measured, with cross-validation between multiple sources to minimize bias. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFRes |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Default Emission factor for emissions from reservoirs |  |  |  |
| Data unit | kgCO2e/MWh |  |  |  |
| Equations referred | Equation 12 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Decision at EB 23, CDM |  |  |  |
| Treatment of uncertainty | Default emission factor uncertainty is addressed by using EB-approved values, which incorporate global average reservoir methane emissions. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Aflooded​ |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Area of land flooded by the reservoir |  |  |  |
| Data unit | Hectares |  |  |  |
| Equations referred | Equation 20 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The area flooded is estimated to using topographic surveys or remote sensing (e.g., satellite imagery). |  |  |  |
| Treatment of uncertainty | Flooded area is estimated using topographic or remote sensing data. Seasonal or measurement variability is considered by applying conservative estimates |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFmethane |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Methane emission factor for the flooded land |  |  |  |
| Data unit | tonCH₄/hectare/year |  |  |  |
| Equations referred | Equation 20 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Assessment Reports or National GHG report inventory |  |  |  |
| Treatment of uncertainty | Methane emission factor is based on IPCC or national inventory values, No uncertainty arises |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | GWPCH4 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global warming potential of methane valid for the relevant commitment period |  |  |  |
| Data unit | t CO2e/t CH4 |  |  |  |
| Equations referred | Equation 20 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 28 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Sixth Assessment Report |  |  |  |
| Treatment of uncertainty | GWP values are sourced from the latest IPCC assessment reports. Uncertainty is minimal, but potential changes in future assessment periods are considered for long-term projections. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Ai |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Area of land use change type |  |  |  |
| Data unit | ha |  |  |  |
| Equations referred | Equation 22 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The area flooded is estimated to using topographic surveys or remote sensing (e.g., satellite imagery). |  |  |  |
| Treatment of uncertainty | Land-use area is measured via surveys or satellite imagery. Seasonal changes or measurement errors are addressed using conservative estimates and cross-verification. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Adis,j |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Area required to continue displaced activity |  |  |  |
| Data unit | ha |  |  |  |
| Equations referred | Equation 23 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The area flooded is estimated to using topographic surveys or remote sensing (e.g., satellite imagery). |  |  |  |
| Treatment of uncertainty | Area for displaced activities is subject to measurement uncertainty; this is minimized using satellite imagery and validated against historical land records. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFi |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for land use type |  |  |  |
| Data unit | tCO2/ha/year |  |  |  |
| Equations referred | Equation 22 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Assessment Reports |  |  |  |
| Treatment of uncertainty | Emission factors for land use are sourced from IPCC reports. Regional variations are addressed through conservative assumptions. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFdis,j |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for land use conversion caused by displacement |  |  |  |
| Data unit | tCO2/ha/year |  |  |  |
| Equations referred | Equation 23 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Assessment Reports or National GHG report inventory |  |  |  |
| Treatment of uncertainty | Emission factors for displaced land use are based on IPCC or national inventories. Uncertainty is mitigated through use of conservative values |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Vdiv,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Volume of water diverted from other uses |  |  |  |
| Data unit | m³/year |  |  |  |
| Equations referred | Equation 18 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Measure abstraction/transfer with calibrated flow meters at intake/outlet points |  |  |  |
| Treatment of uncertainty | Water diversion volumes are measured with calibrated flow meters. Measurement errors are accounted for via regular meter calibration and conservative rounding. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | ECalt,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Energy consumption per unit of water for the alternative supply |  |  |  |
| Data unit | m³/year |  |  |  |
| Equations referred | Equation 18 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determine through manufacturer specifications for pumping systems or actual metered electricity/fuel consumption for comparable systems in the region. Where measured data is not available, use engineering estimates based on pump efficiency, total dynamic head, and flow rate. |  |  |  |
| Treatment of uncertainty | Energy consumption per unit water may vary with pump efficiency. Conservative estimates and manufacturer specifications are used; |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFOM,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Operating Margin Grid Emission Factor |  |  |  |
| Data unit | tCO₂e/MWh |  |  |  |
| Equations referred | Equation 18 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Obtain from the most recent published grid emission factor values provided by the designated national authority, national electricity regulatory body, or UNFCCC-approved grid emission factor database. |  |  |  |
| Treatment of uncertainty | No uncertainty arises as the Operating Margin emission factor is obtained directly from national authorities and UNFCCC-approved databases, reflecting the most accurate and officially recognized value |  |  |  |
| Additional comments | NA |  |  |  |

- Data and parameters monitored

| Data/parameter | EGh,d,yDelivered |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Electricity delivered to grid at hour h, day d, in year y |  |  |  |  |
| Data unit | MWh/hr |  |  |  |  |
| Equations referred | Equation 1, Equation 6 and Equation 12 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement, with automated hourly recording and data logging |  |  |  |
| Measurement methods and procedures | Measured using calibrated electricity meters installed at the grid interconnection point. Readings must be recorded continuously with at least hourly resolution |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent/plant operator (as applicable) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital energy meter (grid-compliant) |  |  |
|  | Accuracy class |  | Class 0.5s or better (as per IEC 62053-22 or equivalent) |  |  |
|  | Calibration requirements |  | - Calibration procedures: As per manufacturer and national grid code. Calibration performed by accredited third-party or utility. - Calibration frequency: As per utility/national regulations. |  |  |
|  | Location |  | At the grid export metering point (typically at the substation or plant interconnection) |  |  |
| QA/QC procedures | Regular calibration and maintenance of meters; comparison with SCADA/utility data; backup logging system in case of data loss. Anomalies flagged and corrected in accordance with approved data handling protocol |  |  |  |  |
| Treatment of uncertainty | No material uncertainty arises as certified grid meters with high accuracy are used and data is cross verified with utility billing. |  |  |  |  |
| Additional comment | Parameter is essential for net emission reduction calculation. |  |  |  |  |

| Data/parameter | EGh,d,yReceived |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Electricity used for pumping water to the upper reservoir at hour h, day d, in year y |  |  |  |  |
| Data unit | MWh/hr |  |  |  |  |
| Equations referred | Equation 14 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement, with automated hourly recording and data logging |  |  |  |
| Measurement methods and procedures | Measured using calibrated electricity meters installed at the grid interconnection point. Readings must be recorded continuously with at least hourly resolution. |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent/plant operator (as applicable) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital energy meter (grid-compliant) |  |  |
|  | Accuracy class |  | Class 0.5s or better (as per IEC 62053-22 or equivalent) |  |  |
|  | Calibration requirements |  | - Calibration procedures: As per manufacturer and national grid code. Calibration performed by accredited third-party or utility. - Calibration frequency: As per utility/national regulations. |  |  |
|  | Location |  | At the grid export metering point (typically at the substation or plant interconnection) |  |  |
| QA/QC procedures | Regular calibration and maintenance of meters; comparison with SCADA/utility data; backup logging system in case of data loss. Anomalies flagged and corrected in accordance with approved data handling protocol |  |  |  |  |
| Treatment of uncertainty | No material uncertainty arises as certified grid meters with high accuracy are used and data is cross verified with utility billing. |  |  |  |  |
| Additional comment | Parameter is essential for net emission reduction calculation. |  |  |  |  |

| Data/parameter | yh,d,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Fraction of delivered electricity by the project activity that is renewable at hour h, day d, in year y |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 1 and Equation 6 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement, with automated hourly recording and data logging |  |  |  |
| Measurement methods and procedures | Fraction is derived from metered renewable generation delivered relative to total generation, using dispatch records. |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent/plant operator (as applicable) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital energy meter (grid-compliant) |  |  |
|  | Accuracy class |  | Class 0.5S or better for energy meters (or equivalent manufacturer specification). |  |  |
|  | Calibration requirements |  | Not applicable. |  |  |
|  | Location |  | At the grid export metering point (typically at the substation or plant interconnection) |  |  |
| QA/QC procedures | Regular calibration and maintenance of meters; comparison with SCADA/utility data; backup logging system in case of data loss. Anomalies flagged and corrected in accordance with approved data handling protocol |  |  |  |  |
| Treatment of uncertainty | No significant uncertainty as the parameter is derived from directly metered data. |  |  |  |  |
| Additional comment | Parameter is essential for net emission reduction calculation. |  |  |  |  |

| Data/parameter | xh,d,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Fraction of received electricity used for pumping water to the upper reservoir that is non-renewable at hour h, day d, in year y |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 14 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement, with automated hourly recording and data logging |  |  |  |
| Measurement methods and procedures | Measured using calibrated electricity meters installed at the grid interconnection point. Readings must be recorded continuously with at least hourly resolution |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent/plant operator (as applicable) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital energy meter (grid-compliant) |  |  |
|  | Accuracy class |  | Class 0.5S or better for energy meters (or equivalent manufacturer specification). |  |  |
|  | Calibration requirements |  | Not applicable. |  |  |
|  | Location |  | At the grid export metering point (typically at the substation or plant interconnection) |  |  |
| QA/QC procedures | Regular calibration and maintenance of meters; comparison with SCADA/utility data; backup logging system in case of data loss. Anomalies flagged and corrected in accordance with approved data handling protocol |  |  |  |  |
| Treatment of uncertainty | No significant uncertainty as the parameter is derived from directly metered data. |  |  |  |  |
| Additional comment | Parameter is essential for net emission reduction calculation. |  |  |  |  |

| Data/parameter | EFDD,h,d,Delivered,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Grid emission factor for delivered electricity at hour h, on day d, in year y (tCO₂/MWh), applicable during the electricity delivery period. |  |  |  |  |
| Data unit | tCO2/hr |  |  |  |  |
| Equations referred | Equation 6 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement with hourly data recording and aggregation |  |  |  |
| Measurement methods and procedures | Emission factor is determined using actual hourly data on electricity generation and fuel-specific CO₂ emissions from grid-connected power plants that are on the margin at each hour. It reflects the emissions from real-time dispatch decisions. Derived from grid operator’s emission factor datasets or approved datasets by national authority/UNFCCC. |  |  |  |  |
| Entity/person responsible for the measurement | Grid operator or designated national/regional authority (e.g., system operator or energy regulator) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Secondary data from grid authority (no direct instrument). |  |  |
|  | Accuracy class |  | Not applicable |  |  |
|  | Calibration requirements |  | Not applicable |  |  |
|  | Location |  | Not applicable |  |  |
| QA/QC procedures | Not Applicable |  |  |  |  |
| Treatment of uncertainty | No uncertainty arises as the parameter is sourced from official datasets. |  |  |  |  |
| Additional comment | Only applicable to delivered electricity. |  |  |  |  |

| Data/parameter | EFDD,h,d,Recived,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Grid emission factor for received electricity for pumping water to the upper reservoir at hour h, on day d, in year y (tCO₂/MWh), applicable during the electricity receiving period. |  |  |  |  |
| Data unit | t CO2/hr |  |  |  |  |
| Equations referred | Equation 14 and Equation 15 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement with hourly data recording and aggregation |  |  |  |
| Measurement methods and procedures | Emission factor is determined using actual hourly data on electricity generation and fuel-specific CO₂ emissions from grid-connected power plants that are on the margin at each hour. It reflects the emissions from real-time dispatch decisions. Derived from grid operator’s emission factor datasets or approved datasets by national authority/UNFCCC. |  |  |  |  |
| Entity/person responsible for the measurement | Grid operator or designated national/regional authority (e.g., system operator or energy regulator) |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Secondary data from grid authority (no direct instrument). |  |  |
|  | Accuracy class |  | Not applicable |  |  |
|  | Calibration requirements |  | Not applicable |  |  |
|  | Location |  | Not applicable |  |  |
| QA/QC procedures | Not Applicable |  |  |  |  |
| Treatment of uncertainty | No uncertainty arises as the parameter is sourced from official datasets. |  |  |  |  |
| Additional comment | Only applicable to delivered electricity. |  |  |  |  |

| Data/parameter | Qf,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Quantity of fuel consumed during year |  |  |  |  |
| Data unit | Gg/yr |  |  |  |  |
| Equations referred | Equation 9 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous or batch-wise (as per operational pattern); aggregated and reported annually |  |  |  |
| Measurement methods and procedures | Fuel consumption data is collected from flow meters, fuel purchase records, or inventory balances. Data should be cross-checked with invoices or utility billing systems and adjusted for storage/stock changes if applicable. |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent or facility operator |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Mass flow meters, fuel tank level meters, or calibrated weighbridges, Data from vendor-certified fuel delivery logs can also be used with cross-verification. |  |  |
|  | Accuracy class |  | Class 1.0 or better |  |  |
|  | Calibration requirements |  | - Procedures: Manufacturer/national standards. - Frequency: As required by standards. |  |  |
|  | Location |  | Fuel delivery point or storage facility. |  |  |
| QA/QC procedures | Cross-check with procurement records and fuel invoices. |  |  |  |  |
| Treatment of uncertainty | Cross-check fuel meter readings with procurement records and inventory reports. Periodic audit of measurement instruments. Documentation of all adjustments (e.g., for spillage, evaporation, or conversion factors) |  |  |  |  |
| Additional comment | Only applicable if fossil fuels are consumed by auxiliary systems. |  |  |  |  |

| Data/parameter | Vevap,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Volume of water lost through evaporation over the year |  |  |  |  |
| Data unit | m3/year |  |  |  |  |
| Equations referred | Equation 15 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Estimated monthly; aggregated annually. |  |  |  |
| Measurement methods and procedures | Estimated using standard hydrological methods such as the water balance method, pan evaporation data |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent or facility operator or hydrological/meteorological authority of the plant |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Measured using evaporation pans, remote sensing, or hydrological balance. |  |  |
|  | Accuracy class |  | As per national hydrological standards or instrument specifications |  |  |
|  | Calibration requirements |  | - Procedures: As per manufacturer/meteorological standards. - Frequency: Annual or as per local standards. |  |  |
|  | Location |  | Reservoir or meteorological station near site. |  |  |
| QA/QC procedures | Cross-check with published climatological data; ensure backup of monitoring logs |  |  |  |  |
| Treatment of uncertainty | Uncertainty addressed by using conservative estimates and validated against regional datasets. |  |  |  |  |
| Additional comment | Applicable only for reservoirs with measurable evaporation. |  |  |  |  |

| Data/parameter | SECpump |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Specific Energy Consumption for Pumping |  |  |  |  |
| Data unit | MWh/m3 |  |  |  |  |
| Equations referred | Equation 15 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous measurement with hourly data logging |  |  |  |
| Measurement methods and procedures | Calculated as the ratio of total electricity consumed for pumping (in MWh) to the volume of water lifted (in m³). Electricity import is measured using a bidirectional digital energy meter, and flow volume is measured using flow meters on the pump discharge line. |  |  |  |  |
| Entity/person responsible for the measurement | Project proponent or facility operator; |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital energy meter for electricity |  |  |
|  | Accuracy class |  | Class 0.5s or better |  |  |
|  | Calibration requirements |  | - Procedures: Manufacturers and national standards. - Frequency: As per manufacturer or national standards. Calibration records must be maintained |  |  |
|  | Location |  | At the grid interconnection or pump motor control center |  |  |
| QA/QC procedures | Cross-verification of energy data with SCADA logs and utility billing |  |  |  |  |
| Treatment of uncertainty | Uncertainty minimized through high-accuracy meters and periodic calibration. |  |  |  |  |
| Additional comment | No Comments |  |  |  |  |

Frequency of submission of monitoring reports

This methodology does not involve activities that result in removals or emission reductions with a risk of reversal. Therefore, the provisions related to the maximum permissible interval between the start date of the crediting period and submission of the first monitoring report, as well as the interval.

- Methodologies principles

Encouraging ambition over time

This Mechanism methodology shall apply increasingly ambitious baselines to encourage ambition of activities over time, while considering host Party circumstances. and should facilitate the deployment of low carbon solutions and technologies.

Mechanism methodologies should facilitate the deployment of technologies or measures that are not widely used or available in specific locations, to facilitate knowledge transfer and encourage deployment of technologies or measures that reduce the cost of decarbonization and unlock investment in low-carbon solutions.

Mechanism methodologies should contain provisions that facilitate the inclusion of progressively more efficient and less greenhouse gas (GHG)-intensive technologies, replicable and scalable mitigation activities, expanding the user base, broader geographic coverage, and greater penetration of low-carbon solutions after initial deployment

In all the approaches of baseline scenario determination, the ambition of the host party is clearly embedded, Over and above the methodology, makes provision for most conservative baseline emission. The difference with BAU and locking-in emissions are quantified.

Contributing to the equitable sharing of mitigation benefits between participating Parties

In this methodology, the conservative approach among all baseline scenarios ensures that emission reductions exceed BAU, directly contributing to the host Party’s NDCs and beyond. The Pumped Hydro Storage project lifetime is ~75 years, 6 times the crediting period, ensuring that a significant portion of emission reductions benefits the host Party beyond the crediting period. Provisions of the methodology incorporate conditions specified by the Designated National Authority (DNA) of the host Party to align with national priorities. Mitigation benefits to the host Party are estimated using conservative assumptions, including adjustments for exogenous factors and crediting period alignment, and are explicitly linked to sustainable development objectives through the application of the SD Tool in project design and implementation. This approach ensures equitable sharing of mitigation benefits between participating Parties while tangibly supporting the host Party’s sustainable development goals.

Implementation and Verification:

- Credit period and project lifetime: The crediting period is clearly defined and shorter than the project lifetime, ensuring continued benefits to the host Party beyond the crediting period.

- DNA provisions: All conditions specified by the host Party’s DNA are incorporated during project design and implementation.

- Sustainable development linkage: Mitigation benefits are tied to SD objectives using the SD Tool; impacts are monitored and reported in accordance with the methodology.

- Conservative estimation of benefits: Emission reductions and associated benefits to the host Party are calculated using conservative assumptions and adjustments for exogenous factors.

- Review and validation: Equitable sharing provisions and SD alignment are verified during validation and each crediting period renewal to ensure compliance and transparency.

- The Designated Operational Entity (DOE) shall verify the equitable sharing provisions, estimation of mitigation benefits, and SD alignment during project validation and verification, as applicable.

Encouraging broad participation

This methodology is designed to encourage broad participation by being accurate, clear, and straightforward, ensuring that a wide range of project participants and host Parties - including those with limited scientific infrastructure or financial capacity - can implement it. It explicitly considers the context on the ground in host Parties, particularly in least developed countries and small island developing States, allowing flexibility to accommodate local conditions. The methodology recognizes and incorporates the knowledge and practices of local communities and Indigenous Peoples where relevant. To address potential data gaps, it permits the use of multiple data sources, conservative default values, and benchmarked data from comparable regions as applicable. Furthermore, the methodology is written in simple, inclusive, gender-sensitive language to ensure accessibility to a wide range of stakeholders, including local communities and Indigenous Peoples, thereby promoting equitable participation in the implementation of Article 6.4 activities

Attributability of emission reductions or net removals to the Article 6.4 activity

For a PHS project, emission reductions are directly and solely attributable to project operation (charging with renewable electricity and displacing fossil-based generation at peak). There’s no significant influence of exogenous factors that would need adjustment, so Section 5 is essentially satisfied by default.

Potential perverse incentives

One potential perverse incentive in Pumped Hydro Storage (PHS) projects is the possibility of altering the storage reservoir volume after validation. Increasing the storage capacity post-validation would allow the project to shift larger amounts of electricity from off-peak to peak periods. This could artificially increase the calculated emission reductions beyond what was originally validated, leading to over-crediting.

Avoidance Measure in the Methodology:

- To address this risk, the methodology requires that the storage reservoir volume, turbine capacity, and operational design parameters be fixed at the time of validation and remain unchanged throughout the crediting period, except for normal operational degradation or maintenance. Any modification to increase reservoir capacity post-validation shall render the project ineligible for claiming additional emission reductions under this methodology.

This safeguard ensures that emission reductions are attributed solely to the validated project design and not to post-validation alterations.

Rebound effects

The issue of rebound effects is not relevant for the type of activity covered by this methodology. Rebound effects typically refer to an increase in the overall level of service (e.g., more electricity consumed) because of improved efficiency or reduced energy cost. This methodology involves time-shifting of renewable electricity via pumped storage hydropower (PHS) and does not lead to increased electricity service levels, for the following reasons:

  - No increase in total electricity demand or service

- The project activity stores renewable electricity during low-demand or surplus periods (typically daytime) and releases it during peak demand periods (typically at night), thereby displacing fossil-based generation. This shift does not increase overall electricity consumption or supply but rather alters its timing.

  - Nighttime supply replaces fossil generation

- In the absence of this project, nighttime electricity demand would be largely met by coal or gas-based power plants. By supplying stored renewable electricity at night, the project reduces the carbon intensity of the grid during those hours. This is substitution, not expansion of service, and therefore does not lead to a rebound effect.

  - No price signal that encourages overuse

- PHS projects do not reduce electricity tariffs or cause energy prices to drop in a way that would incentivize end users to increase consumption. The cost of storage and renewable procurement remains relatively high, limiting any potential demand-side increase.

- - - - -

Document information

| Version | Date | Description |
| --- | --- | --- |
| 0.2.0 | 19 August 2025 | Revision to incorporate new sections and sub-sections in line with current standards; provide completion instructions, realign their sequence, and allow inclusion of explanatory notes. |
| 01.0 | 18 December 2024 | Initial publication of form template. |
| Decision Class: Regulatory Document Type: Form Business Function: Methodology Keywords: A6.4 mechanism, developing methodologies and tools |  |  |

- Format for footnotes.